§

INTERNATIONAL  TELECOMMUNICATION  UNION

CCITT	H.261

THE  INTERNATIONAL
TELEGRAPH  AND  TELEPHONE
CONSULTATIVE  COMMITTEE

LINE  TRANSMISSION
ON  NON-TELEPHONE  SIGNALS

VIDEO  CODEC  FOR  AUDIOVISUAL  SERVICES
AT  p  ×   64  kbit/s

Recommendation  H.261

§

Geneva, 1990

# FOREWORD

The CCITT (the International Telegraph and Telephone Consultative Committee) is the permanent organ of the International Telecommunication Union (ITU). CCITT is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The Plenary Assembly of CCITT which meets every four years, establishes the topics for study and approves Recommendations prepared by its Study Groups. The approval of Recommendations by the members of CCITT between Plenary Assemblies is covered by the procedure laid down in CCITT Resolution No. 2 (Melbourne, 1988).

Recommendation H.261 was prepared by Study Group XV and was approved under the Resolution No. 2 procedure on the 14 of December 1990.

___________________

# CCITT  NOTE

In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication Administration and a recognized private operating agency.

ã  ITU  1990

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

PAGE BLANCHE

Recommendation H.261

Recommendation H.261

VIDEO CODEC FOR AUDIOVISUAL SERVICES AT p ´ 64 kbit/s

(revised 1990)

The CCITT,

considering

(a)	that there is significant customer demand for videophone, videoconference and other audiovisual services;

(b)	that circuits to meet this demand can be provided by digital transmission using the B, H0 rates or their multiples up to the primary rate or H11/H12 rates;

(c)	that ISDNs are likely to be available in some countries that provide a switched transmission service at the B, H0 or H11/H12 rate;

(d)	that the existence of different digital hierarchies and different television standards in different parts of the world complicates the problems of specifying coding and transmission standards for international connections;

(e)	that a number of audiovisual services are likely to appear using basic and primary rate ISDN accesses and that some means of intercommunication among these terminals should be possible;

(f)	that the video codec provides an essential element of the infrastructure for audiovisual services which allows such intercommunication in the framework of Recommendation H.200;

(g)	that Recommendation H.120 for videoconferencing using primary digital group transmission was the first in an evolving series of Recommendations,

appreciating

that advances have been made in research and development of video coding and bit rate reduction techniques which lead to the use of lower bit rates down to 64 kbit/s so that this may be considered as the second in the evolving series of Recommendations,

and noting

that it is the basic objective of the CCITT to recommend unique solutions for international connections,

recommends

that in addition to those codecs complying to Recommendation H.120, codecs having signal processing and transmission coding characteristics described below should be used for international audiovisual services.

Note 1 — Codecs of this type are also suitable for some television services where full broadcast quality is not required.

Note 2 — Equipment for transcoding from and to codecs according to Recommendation H.120 is under study.

# 1	Scope

This Recommendation describes the video coding and decoding methods for the moving picture component of audiovisual services at the rates of p ´ 64 kbit/s, where p is in the range 1 to 30.

# 2	Brief specification

An outline block diagram of the codec is given in Figure 1/H.261.

FIGURE 1/H.261

## 2.1	Video input and output

To permit a single Recommendation to cover use in and between regions using 625- and 525-line television standards, the source coder operates on pictures based on a common intermediate format (CIF). The standards of the input and output television signals, which may, for example, be composite or component, analogue or digital and the methods of performing any necessary conversion to and from the source coding format are not subject to Recommendation.

## 2.2	Digital output and input

The video coder provides a self-contained digital bit stream which may be combined with other multi-facility signals (for example as defined in Recommendation H.221). The video decoder performs the reverse process.

## 2.3	Sampling frequency

Pictures are sampled at an integer multiple of the video line rate.  This sampling clock and the digital network clock are asynchronous.

## 2.4	Source coding algorithm

A hybrid of inter-picture prediction to utilize temporal redundancy and transform coding of the remaining signal to reduce spatial redundancy is adopted. The decoder has motion compensation capability, allowing optional incorporation of this technique in the coder.

## 2.5	Bit rate

This Recommendation is primarily intended for use at video bit rates between approximately 40 kbit/s and 2 Mbit/s.

## 2.6	Symmetry of transmission

The codec may be used for bidirectional or unidirectional visual communication.

## 2.7	Error handling

The transmitted bit-stream contains a BCH1) (511,493) forward error correction code. Use of this by the decoder is optional.

## 2.8	Multipoint operation

Features necessary to support switched multipoint operation are included.

# 3	Source coder

## 3.1	Source format

The source coder operates on non-interlaced pictures occurring 30 000/1001 (approximately 29.97) times per second. The tolerance on picture frequency is ± 50 ppm.

Pictures are coded as luminance and two colour difference components (Y, CB and CR). These components and the codes representing their sampled values are as defined in CCIR Recommendation 601.

Black = 16

White = 235

Zero colour difference = 128

Peak colour difference = 16 and 240.

These values are nominal ones and the coding algorithm functions with input values of 1 through to 254.

Two picture scanning formats are specified.

In the first format (CIF), the luminance sampling structure is 352 pels per line, 288 lines per picture in an orthogonal arrangement. Sampling of each of the two colour difference components is at 176 pels per line, 144 lines per picture, orthogonal. Colour difference samples are sited such that their block boundaries coincide with luminance block boundaries as shown in Figure 2/H.261. The picture area covered by these numbers of pels and lines has an aspect ratio of 4:3 and corresponds to the active portion of the local standard video input.

Note — The number of pels per line is compatible with sampling the active portions of the luminance and colour difference signals from 525- or 625-line sources at 6.75 and 3.375 MHz respectively. These frequencies have a simple relationship to those in CCIR Recommendation 601.

FIGURE 2/H.261

The second format, quarter-CIF (QCIF), has half the number of pels and half the number of lines stated above. All codecs must be able to operate using QCIF. Some codecs can also operate with CIF.

Means shall be provided to restrict the maximum picture rate of encoders by having at least 0, 1, 2 or 3 non-transmitted pictures between transmitted ones. Selection of this minimum number and CIF or QCIF shall be by external means (for example via Recommendation H.221).

## 3.2	Video source coding algorithm

The source coder is shown in generalized form in Figure 3/H.261. The main elements are prediction, block transformation and quantization.

The prediction error (INTER mode) or the input picture (INTRA mode) is subdivided into 8 pel by 8 line blocks which are segmented as transmitted or non-transmitted. Further, four luminance blocks and the two spatially corresponding colour difference blocks are combined to form a macroblock as shown in Figure 10/H.261.

The criteria for choice of mode and transmitting a block are not subject to recommendation and may be varied dynamically as part of the coding control strategy. Transmitted blocks are transformed and resulting coefficients are quantized and variable length coded.

### 3.2.1	Prediction

The prediction is inter-picture and may be augmented by motion compensation (see § 3.2.2) and a spatial filter (see § 3.2.3).

FIGURE 3/H.261

### 3.2.2	Motion compensation

Motion compensation (MC) is optional in the encoder. The decoder will accept one vector per macroblock. Both horizontal and vertical components of these motion vectors have integer values not exceeding ± 15. The vector is used for all four luminance blocks in the macroblock. The motion vector for both colour difference blocks is derived by halving the component values of the macroblock vector and truncating the magnitude parts towards zero to yield integer components.

A positive value of the horizontal or vertical component of the motion vector signifies that the prediction is formed from pels in the previous picture which are spatially to the right or below the pels being predicted.

Motion vectors are restricted such that all pels referenced by them are within the coded picture area.

### 3.2.3	Loop filter

The prediction process may be modified by a two-dimensional spatial filter (FIL) which operates on pels within a predicted 8 by 8 block.

The filter is separable into one-dimensional horizontal and vertical functions. Both are non-recursive with coefficients of 1/4, 1/2, 1/4 except at block edges where one of the taps would fall outside the block. In such cases the 1-D filter is changed to have coefficients of 0, 1, 0. Full arithmetic precision is retained with rounding to 8 bit integer values at the 2-D filter output. Values whose fractional part is one half are rounded up.

The filter is switched on/off for all six blocks in a macroblock according to the macroblock type (see § 4.2.3 MTYPE).

### 3.2.4	Transformer

Transmitted blocks are first processed by a separable two-dimensional discrete cosine transform of size 8 by 8. The output from the inverse transform ranges from -256 to +255 after clipping to be represented with 9 bits. The transfer function of the inverse transform is given by:

µ

§

with u, v, x, y = 0, 1, 2, . . ., 7

where	x,y = spatial coordinates in the pel domain,

u,v 	= coordinates in the transform domain,

C(u) =  for u = 0, otherwise 1,

C(v) =  for v = 0, otherwise 1.

Note — Within the block being transformed, x = 0 and y = 0 refer to the pel nearest the left and top edges of the picture respectively.

The arithmetic procedures for computing the transforms are not defined, but the inverse one should meet the error tolerance specified in Annex A.

### 3.2.5	Quantization

The number of quantizers is 1 for the INTRA dc coefficient and 31 for all other coefficients. Within a macroblock the same quantizer is used for all coefficients except the INTRA dc one. The decision levels are not defined. The INTRA dc coefficient is nominally the transform value linearly quantized with a stepsize of 8 and no dead-zone. Each of the other 31 quantizers is also nominally linear but with a central dead-zone around zero and with a step size of an even value in the range 2 to 62.

The reconstruction levels are as defined in § 4.2.4.

Note — For the smaller quantization step sizes, the full dynamic range of the transform coefficients cannot be represented.

### 3.2.6	Clipping of reconstructed picture

To prevent quantization distortion of transform coefficient amplitudes causing arithmetic overflow in the encoder and decoder loops, clipping functions are inserted. The clipping function is applied to the reconstructed picture which is formed by summing the prediction and the prediction error as modified by the coding process. This clipper operates on resulting pel values less than 0 or greater than 255, changing them to 0 and 255 respectively.

## 3.3	Coding control

Several parameters may be varied to control the rate of generation of coded video data. These include processing prior to the source coder, the quantizer, block significance criterion and temporal subsampling. The proportions of such measures in the overall control strategy are not subject to recommendation.

When invoked, temporal subsampling is performed by discarding complete pictures.

## 3.4	Forced updating

This function is achieved by forcing the use of the INTRA mode of the coding algorithm. The update pattern is not defined. For control of accumulation of inverse transform mismatch error a macroblock should be forcibly updated at least once per every 132 times it is transmitted.

# 4	Video multiplex coder

## 4.1	Data structure

Unless specified otherwise the most significant bit is transmitted first. This is bit 1 and is the leftmost bit in the code tables in this Recommendation. Unless specified otherwise all unused or spare bits are set to “1”. Spare bits must not be used until their functions are specified by the CCITT.

## 4.2	Video multiplex arrangement

The video multiplex is arranged in a hierarchical structure with four layers. From top to bottom the layers are:

—	Picture.

—	Group of blocks (GOB).

—	Macroblock (MB).

—	Block.

A syntax diagram of the video multiplex coder is shown in Figure 4/H.261. Abbreviations are defined in later sections.

FIGURE 4/H.261

### 4.2.1	Picture layer

Data for each picture consists of a picture header followed by data for GOBs. The structure is shown in Figure 5 /H.261. Picture headers for dropped pictures are not transmitted.

4.2.1.1	Picture Start Code (PCS) (20 bits)

µ

PSC

TR

PTYPE

PEI

PSPARE

PEI

GOB Data

FIGURE 5/H.261

Structure of picture layer

§

A word of 20 bits. Its value is 0000 0000 0000 0001 0000.

#### 4.2.1.2	Temporal Reference (TR) (5 bits)

A 5-bit number which can have 32 possible values. It is formed by incrementing its value in the previously transmitted picture header by one plus the number of non-transmitted pictures (at 29.97 Hz) since that last transmitted one. The arithmetic is performed with only the five LSBs.

#### 4.2.1.3	Type Information (PTYPE) (6 bits)

Information about the complete picture:

Bit 1		Split screen indicator, “0” off, “1” on.

Bit 2		Document camera indicator. “0” off, “1” on.

Bit 3		Freeze Picture Release. “0” off, “1” on.

Bit 4		Source Format. “0” QCIF, “1” CIF.

Bits 5 to 6	Spare.

#### 4.2.1.4	Extra Insertion Information (PEI) (1 bit)

A bit which when set to “1” signals the presence of the following optional data field.

#### 4.2.1.5	Spare Information (PSPARE) (0/8/16 . . . bits)

If PEI is set to “1”, then 9 bits follow consisting of 8 bits of data (PSPARE) and then another PEI bit to indicate if a further 9 bits follow and so on. Encoders must not insert PSPARE until specified by the CCITT. Decoders must be designed to discard PSPARE if PEI is set to 1. This will allow the CCITT to specify future backward compatible additions in PSPARE.

### 4.2.2	Group of blocks layer

Each picture is divided into groups of blocks (GOBs). A group of blocks (GOB) comprises one twelfth of the CIF or one third of the QCIF picture areas (see Figure 6/H.261). A GOB relates to 176 pels by 48 lines of Y and the spatially corresponding 88 pels by 24 lines of each of CB and CR.

Data for each group of blocks consists of a GOB header followed by data for macroblocks. The structure is shown in Figure 7/H.261. Each GOB header is transmitted once between picture start codes in the CIF or QCIF sequence numbered in Figure 6/H.261, even if no macroblock data is present in that GOB.

µ

1

2

1

3

4

3

5

6

5

7

8

QCIF

9

10

11

12

CIF

FIGURE 6/H.261

Arrangement of GOBs in a picture

§

#### 4.2.2.1	Group of blocks start code (GBSC) (16 bits)

A word of 16 bits, 0000 0000 0000 0001.

4.2.2.2	Group number (GN) (4 bits)

µ

GBSC

GN

GQUANT

GEI

GSPARE

GEI

MB Data

FIGURE 7/H.261

Structure of group of blocks layer

§

Four bits indicating the position of the group of blocks. The bits are the binary representation of the number in Figure 6/H.261. Group numbers 13, 14 and 15 are reserved for future use. Group number 0 is used in the PSC.

#### 4.2.2.3	Quantizer Information (GQUANT) (5 bits)

A fixed length codeword of 5 bits which indicates the quantizer to be used in the group of blocks until overridden by any subsequent MQUANT. The codewords are the natural binary representations of the values of QUANT (§ 4.2.4) which, being half the step sizes, range from 1 to 31.

#### 4.2.2.4	Extra insertion information (GEI) (1 bit)

A bit which when set to “1” signals the presence of the following optional data field.

#### 4.2.2.5	Spare information (GSPARE) (0/8/16 . . . bits)

If GEI is set to “1”, then 9 bits follow consisting of 8 bits of data (GSPARE) and then another GEI bit to indicate if a further 9 bits follow and so on. Encoders must not insert GSPARE until specified by the CCITT. Decoders must be designed to discard GSPARE if GEI is set to 1. This will allow the CCITT to specify future “backward” compatible additions in GSPARE.

Note — Emulation of start codes may occur if the future specification of GSPARE has no restrictions on the final GSPARE data bits.

### 4.2.3	Macroblock layer

Each GOB is divided into 33 macroblocks as shown in Figure 8/H.261. A macroblock relates to 16 pels by 16 lines of Y and the spatially corresponding 8 pels by 8 lines of each of CB and CR.

Data for a macroblock consists of a MB header followed by data for blocks (see Figure 9/H.261). MQUANT, MVD and CBP are present when indicated by MTYPE.

µ

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

FIGURE 8/H.261

Arrangement of macroblocks in a GOB

§

4.2.3.1	Macroblock address (MBA) (Variable length)

µ

MBA

MTYPE

MQUANT

MVD

CBP

Block data

FIGURE 9/H.261

Structure of macroblock layer

§

A variable length codeword indicating the position of a macroblock within a group of blocks. The transmission order is as shown in Figure 8/H.261. For the first transmitted macroblock in a GOB, MBA is the absolute address in Figure 8/H.261. For subsequent macroblocks, MBA is the difference between the absolute addresses of the macroblock and the last transmitted macroblock. The code table for MBA is given in Table 1/H.261.

An extra codeword is available in the table for bit stuffing immediately after a GOB header or a coded macroblock (MBA stuffing). This codeword should be discarded by decoders.

The VLC for start code is also shown in Table 1/H.261.

MBA is always included in transmitted macroblocks.

µTABLE 1/H.261

VLC table for macroblock addressing

MBA

Code

MBA

Code

01

1000

17

0000  0101  1000

02

0110

18

0000  0101  0100

03

0100

19

0000  0101  0000

04

0011

20

0000  0100  1100

05

0010

21

0000  0100  1000

06

0001

1

22

0000  0100  0110

07

0001

0

23

0000  0100  0100

08

0000

111

24

0000  0100  0010

09

0000

110

25

0000  0100  0000

10

0000

1011

26

0000  0011  1110

11

0000

1010

27

0000  0011  1100

12

0000

1001

28

0000  0011  1010

13

0000

1000

29

0000  0011  1000

14

0000

0111

30

0000  0011  0110

15

0000

0110

31

0000  0011  0100

16

0000

0101 11

32

0000  0011  0010

33

0000  0011  0000

MBA stuffing

0000  0001  1110

Start code

0000000000  0000  0000  00010

§

Macroblocks are not transmitted when they contain no information for that part of the picture.

#### 4.2.3.2	Type information (MTYPE) (Variable length)

Variable length codewords giving information about the macroblock and which data elements are present. Macroblock types, included elements and VLC words are listed in Table 2/H.261.

MTYPE is always included in transmitted macroblocks.

4.2.3.3	Quantizer (MQUANT) (5 bits)

µTABLE 2/H.261

VLC table for MTYPE

Prediction

MQUANT

MVD

CBP

TCOEFF

VLC

Intra

x

0001

Intra

x

x

0000 001

Inter

x

x

1

Inter

x

x

x

0000 1

Inter + MC

x

0000 0000 10

Inter + MC

x

x

x

0000 0001

Inter + MC

x

x

x

x

0000 0000 01

Inter + MC + FIL

x

001

Inter + MC + FIL

x

x

x

01

Inter + MC + FIL

x

x

x

x

0000 01

Note 1 — “x” means that the item is present in the macroblock.

Note 2 — It is possible to apply the filter in a non-motion compensated macroblock by declaring it as MC + FIL but with a zero vector.

§

MQUANT is present only if so indicated by MTYPE.

A codeword of 5 bits signifying the quantizer to be used for this and any following blocks in the group of blocks until overridden by any subsequent MQUANT.

Codewords for MQUANT are the same as for GQUANT.

#### 4.2.3.4	Motion vector data (MVD) (Variable length)

Motion vector data is included for all MC macroblocks. MVD is obtained from the macroblock vector by subtracting the vector of the preceding macroblock. For this calculation the vector of the preceding macroblock is regarded as zero in the following three situations:

1)	evaluating MVD for macroblocks 1, 12 and 23;

2)	evaluating MVD for macroblocks in which MBA does not represent a difference of 1;

3)	MTYPE of the previous macroblock was not MC.

MVD consists of a variable length codeword for the horizontal component followed by a variable length codeword for the vertical component. Variable length codes are given in Table 3/H.261.

Advantage is taken of the fact that the range of motion vector values is constrained. Each VLC word represents a pair of difference values. Only one of the pair will yield a macroblock vector falling within the permitted range.

#### 4.2.3.5	 Coded block pattern (CBP) (Variable length)

CBP is present if indicated by MTYPE. The codeword gives a pattern number signifying those blocks in the macroblock for which at least one transform coefficient is transmitted. The pattern number is given by:

32 · P1 + 16 · P2 + 8 · P3 + 4 · P4 + 2 · P5 + P6

where Pn = 1 if any efficient is present for block n, else 0. Block numbering is given in Figure 10/H.261.

The codewords for CBP are given in Table 4/H.261.

4.2.4	Block layer

µTABLE 3/H.261

VLC table for MVD

MVD

Code

-16 &  16

0000  0011  001

-15 &  17

0000  0011  011

-14 &  18

0000  0011  101

-13 &  19

0000  0011  111

-12 &  20

0000  0100  001

-11 &  21

0000  0100  011

-10 &  22

0000  0100  110

0-9 &  23

0000  0101  010

0-8 &  24

0000  0101  110

0-7 &  25

0000  0111  00o

0-6 &  26

0000  1001  101

0-5 &  27

0000  1011  000

0-4 &  28

0000  111 0 000

0-3 &  29

0001  1000  000

0-2 &  30

0011  0000  000

0-100000

01100  000  000

000000

10000  000 0000

110000

01000  0000  00

000002 & -3000

0010  000000  0

0003 & -29

0001  0000  000

0004 & -28

0000  1100  000

0005 & -27

0000  1010  000

0006 & -26

0000  1000  000

0007 & -25

0000  0110  000

0008 & -24

0000  0101  100

0009 & -23

0000  0101  000

0010 & -22

0000  0100  100

0011 & -21

0000  0100  010

0012 & -20

0000  0100  000

0013 & -19

0000  0011  110

0014 & -18

0000  0011  100

0015 & -17

0000  0011  010

§

µTABLE 4/H.261

VLC table for CBP

CBP

Code

CBP

Code

60

1110  oooo

35

0001  1100  0

04

1101  0000

13

0001  1011  0

08

1100  0000

49

0001  1010  0

16

1011  0000

21

0001  1001  0

32

1010  0000

41

0001  1000  0

12

1001  1000

14

0001  0111  0

48

1001  0000

50

0001  0110  0

20

1000  1000

22

0001  0101  0

40

1000  0000

42

0001  0100  0

28

0111  1000

15

0001  0011  0

44

0111  0000

51

0001  0010  0

52

0110  1000

23

0001  0001  0

56

0110  0000

43

0001  0000  0

01

0101  1000

25

0000  1111  0

61

0101  0000

37

0000  1110  0

02

0100  1000

26

0000  1101  0

62

0100  0000

38

0000  1100  0

24

0011  1100

29

0000  1011  0

36

0011  1000

45

0000  1010  0

03

0011  0100

53

0000  1001  0

63

0011  0000

57

0000  1000  0

05

0010  1110

30

0000  0111  0

99

0010  1100

46

0000  0110  0

17

0010  1010

54

0000  0101  0

33

0010  1000

58

0000  0100  0

96

0010  0110

31

0000  0011  1

10

0010  0100

47

0000  0011  0

18

0010  0010

55

0000  0010  1

34

0010  0000

59

0000  0010  0

97

0001  1111

27

0000  0001  1

11

0001  1110

39

0000  0001  0

19

0001  1101

§

A macroblock comprises four luminance blocks and one of each of the two colour difference blocks (see Figure 10/H.261).

Data for a block consists of codewords for transform coefficients followed by an end of block marker (see Figure 11/H.261). The order of block transmission is as in Figure 10/H.261.

4.2.4.1	Transform coefficients (TCOEFF)

µ

1

2

5

6

3

4

Y

CB

CR

FIGURE 10/H.261

Arrangement of blocks in a macroblock

§

µ

TCOEFF

EOB

FIGURE 11/H.261

Structure of block layer

§

Transform coefficient data is always present for all six blocks in a macroblock when MTYPE indicates INTRA. In other cases MTYPE and CBP signal which blocks have coefficient data transmitted for them. The quantized transform coefficients are sequentially transmitted according to the sequence given in Figure 12/H.261.

µ

1

2

6

7

15

16

28

29

¾¾®	Increasing cycles per

3

5

8

14

17

27

30

43

½	picture width

4

9

13

18

26

31

42

44

¯

10

12

19

25

32

41

45

54

Increasing cycles per

11

20

24

33

40

46

53

55

picture height

21

23

34

39

47

52

56

61

22

35

38

48

51

57

60

62

36

37

49

50

58

59

63

64

FIGURE 12/H.261

Transmission order for transform coefficients

§

The most commonly occurring combinations of successive zeros (RUN) and the following value (LEVEL) are encoded with variable length codes. Other combinations of (RUN, LEVEL) are encoded with a 20-bit word consisting of 6 bits ESCAPE, 6 bits RUN and 8 bits LEVEL. For the variable length encoding there are two code tables, one being used for the first transmitted LEVEL in INTER, INTER+MC and INTER+MC+FIL blocks, the second for all other LEVELs except the first one in INTRA blocks which is fixed length coded with 8 bits.

Codes are given in Table 5/H.261.

The most commonly occurring combinations of zero-run and the following value are encoded with variable length codes as listed in the table below. End of block (EOB) is in this set. Because CBP indicates those blocks with no coefficient data, EOB cannot occur as the first coefficient. Hence EOB can be removed from the VLC table for the first coefficient.

The last bit “s” denotes the sign of the level, “0” for positive and “1” for negative.

µ

TABLE 5/H.261

VLC table for TCOEFF

Run

Level

Code

EOB

10

0

1

1sa)  If first coefficient in block

0

1

11s  Not first coefficient in  block

0

2

0100  s

0

3

0010  1s

0

4

0000  110s

0

5

0010  0110  s

0

6

0010  0001  s

0

7

0000  0010  10s

0

8

0000  0001  1101  s

0

9

0000  0001  1000  s

0

10

0000  0001  0011  s

0

11

0000  0001  0000  s

0

12

0000  0000  1101  0s

0

13

0000  0000  1100  1s

0

14

0000  0000  1100  0s

0

15

0000  0000  1011  1s

1

1

011s

1

2

0001  10s

1

3

0010  0101  s

1

4

0000  0011  00s

1

5

0000  0001  1011  s

1

6

0000  0000  1011  0s

1

7

0000  0000  1010  1s

a)	Never used in INTRA macroblocks.

TABLE 5/H.261 (Cont.)

Run

Level

Code

2

1

0101  s

2

2

0000  100s

2

3

0000  0010  11s

2

4

0000  0001  0100  s

2

5

0000  0000  1010  0s

3

1

0011  1s

3

2

0010  0100  s

3

3

0000  0001  1100  s

3

4

0000  0000  1001  1s

4

1

0011  0s

4

2

0000  0011  11s

4

3

0000  0001  0010  s

5

1

0001  11s

5

2

0000  0010  01s

5

3

0000  0000  1001  0s

6

1

0001  01s

6

2

0000  0001  1110  s

7

1

0001  00s

7

2

0000  0001  0101  s

8

1

0000  111s

8

2

0000  0001  0001  s

9

1

0000  101s

9

2

0000  0000  1000  1s

10

1

0010  0111  s

10

2

0000  0000  1000  0s

11

1

0010  0011  s

12

1

0010  0010  s

13

1

0010  0000  s

14

1

0000  0011  10s

15

1

0000  0011  01s

16

1

0000  0010  00s

17

1

0000  0001  1111  s

18

1

0000  0001  1010  s

19

1

0000  0001  1001  s

20

1

0000  0001  0111  s

21

1

0000  0001  0110  s

22

1

0000  0000  1111  1s

23

1

0000  0000  1111  0s

24

1

0000  0000  1110  1s

25

1

0000  0000  1110  0s

26

1

0000  0000  1101  1s

Escape

0000  01

The remaining combinations of (run, level) are encoded with a 20-bit word consisting of 6 bits escape 6 bits run and 8 bits level. Use of this 20-bit word form encoding the combinations listed in the VLC table is not prohibited.

Run is a 6 bit fixed length code

Level is an 8 bit fixed length code

Run

Code

Level

Code

0

0000  00

-128

FORBIDDEN

1

0000  01

-127

1000  0001

2

0000  10

×

×

×

×

00-2

1111  1110

×

×

00-1

1111  1111

63

1111  11

0000

FORBIDDEN

0001

0000  0001

0002

0000  0010

×

×

0127

0111  1111

For all coefficients other than the INTRA dc one, the reconstruction levels (REC) are in the range -2048 to 2047 and are given by clipping the results of the following formules:

REC = 0; level = 0

Note — QUANT ranges from 1 to 31 and is transmitted by either GQUANT or MQUANT.

Reconstruction levels (REC)

Level

1

2

3

4

×

QUANT
8

9

×

17

18

×

30

31

-127

-255

-509

-765

-1019

×

-2039

-2048

×

-2048

-2048

×

-2048

-2048

-126

-253

-505

-759

-1011

×

-2023

-2048

×

-2048

-2048

×

-2048

-2048

×

×

×

×

×

×

×

×

×

×

×

×

×

×

00-2

00-5

00-9

0-15

00-19

×

00-39

00-45

×

00-85

00-89

×

0-149

00-155

00-1

00-3

00-5

00-9

00-11

×

00-23

00-27

×

00-51

00-53

×

00-89

00-93

0000

0000

0000

0000

00000

×

00000

00000

×

00000

00000

×

00000

00000

0001

0003

0005

0009

00011

×

00023

00027

×

00051

00053

×

00089

00093

0002

0005

0009

0015

00019

×

00039

00045

×

00085

00089

×

00149

00155

0003

0007

0013

0021

00027

×

00055

00063

×

00119

00125

×

00209

00217

0004

0009

0017

0027

00035

×

00071

00081

×

00153

00161

×

00269

00279

0005

0011

0021

0033

00043

×

00087

00099

×

00187

00197

×

00329

00341

×

×

×

×

×

×

×

×

×

×

×

×

×

×

0056

0113

0225

0339

00451

×

00903

01017

×

01921

02033

×

02047

02047

0057

0115

0229

0345

00459

×

00919

01035

×

01955

02047

×

02047

02047

0058

0117

0233

0351

00467

×

00935

01053

×

01989

02047

×

02047

02047

0059

0119

0237

0357

00475

×

00951

01071

×

02023

02047

×

02047

02047

0060

0121

0241

0363

00483

×

00967

01089

×

02047

02047

×

02047

02047

×

×

×

×

×

×

×

×

×

×

×

×

×

×

0125

0251

0501

0753

01003

×

02007

02047

×

02047

02047

×

02047

02047

126

0253

0505

0759

01011

×

02023

02047

×

02047

02047

×

02047

02047

127

0255

0509

0765

01019

×

02039

02047

×

02047

02047

×

02047

02047

Note — Reconstruction levels are symmetrical with respect to the sign of level except for 2047/-2048.

For INTRA blocks the first coefficient is nominally the transform dc  value linearly quantized with a step size of 8 and no dead-zone. The resulting values are represented with 8 bits. A nominally black block will give 0001 0000 and a nominally white one 1110 1011. The code 0000 0000 is not used. The code 1000 0000 is not used, the reconstruction level of 1024 being coded as 1111 1111 (see Table 6/H.261).

§

Coefficients after the last non-zero one are not transmitted. EOB (end of block code) is always the last item in blocks for which coefficients are transmitted.

## 4.3	Multipoint considerations

The following facilities are provided to support switched multipoint operation.

### 4.3.1	Freeze picture request

Causes the decoder to freeze its displayed picture until a freeze picture release signal is received or a timeout period of at least six seconds has expired. The transmission of this signal is via external means (for example by Recommendation H.221).

µTABLE 6/H.261

Reconstruction levels for INTRA-mode dc coefficient

FLC

Reconstruction level into
inverse transform

0000  0001     (1)

0008

0000  0010     (2)

0016

0000  0011     (3)

0024

×

×

×

×

0111  1111  (127)

1016

1111  1111  (255)

1024

1000  0001  (129)

1032

×

×

×

×

1111  1101  (253)

2024

1111  1110  (254)

2032

Note — The decoded value corresponding to FLC “n” is 8n except FLC 255 gives 1024.

§

### 4.3.2	Fast update request

Causes the encoder to encode its next picture in INTRA mode with coding parameters such as to avoid buffer overflow. The transmission method for this signal is via external means (for example by Recommendation H.221).

### 4.3.3	Freeze picture release

A signal from an encoder which has responded to a fast update request and allows a decoder to exit from its freeze picture mode and display decoded pictures in the normal manner. This signal is transmitted by bit 3 of PTYPE (see § 4.2.1) in the picture header of the first picture coded in response to the fast update request.

# 5	Transmission coder

## 5.1	Bit rate

The transmission clock is provided externally (for example from an I.420 interface).

## 5.2	Video data buffering

The encoder must control its output bitstream to comply with the requirements of the hypothetical reference decoder defined in Annex B.

When operating with CIF the number of bits created by coding any single picture must not exceed 256 · K bits. K = 1024.

When operating with QCIF the number of bits created by coding any single picture must not exceed 64 · K bits.

In both the above cases the bit count includes the picture start code and all other data related to that picture including PSPARE, GSPARE and MBA stuffing. The bit count does not include error correction framing bits, fill indicator (Fi), fill bits or error correction parity information described in § 5.4 below.

Video data must be provided on every valid clock cycle. This can be ensured by the use of either the fill bit indicator (Fi) and subsequent fill all 1's bits in the error corrector block framing (see Figure 13/H.261) or MBA stuffing (§ 4.2.3) or both.

FIGURE 13/H.261

## 5.3	Video coding delay

This item is included in this Recommendation because the video encoder and video decoder delays need to be known to allow audio compensation delays to be fixed when H.261 is used to form part of a conversational service. This will allow lip synchronization to be maintained. Annex C recommends a method by which the delay figures are established. Other delay measurement methods may be used but they must be designed in a way to produce similar results to the method given in Annex C.

## 5.4	Forward error correction for coded video signal

### 5.4.1	Error correcting code

The transmitted bitstream contains a BCH (511,493) forward error correction code. Use of this by the decoder is optional.

### 5.4.2	Generator polynomial

g(x) = (x9 + x4 + 1)(x9 + x6 + x4 + x3 + 1)

Example: for the input data of “01111 . . . 11” (493 bits) the resulting correction parity bits are “011011010100011011” (18 bits).

### 5.4.3	Error correction framing

To allow the video data and error correction parity information to be identified by a decoder an error correction framing pattern is included. This consists of a multiframe of eight frames, each frame comprising 1 bit framing, 1 bit fill indicator (Fi), 492 bits of coded data (or fill all 1s) and 18 bits parity.  The frame alignment pattern is:

(S1S2S3S4S5S6S7S8) = (00011011).

See Figure 13/H.261 for the frame arrangement. The parity is calculated against the 493-bits including fill indicator (Fi).

The fill indicator (Fi) can be set to zero by an encoder. In this case only 492 consecutive fill bits (fill all 1s) plus parity are sent and no coded data is transmitted. This may be used to meet the requirement in § 5.2 to provide video data on every valid clock cycle.

#### 5.4.4	Relock time for error corrector framing

Three consecutive error correction framing sequences (24 bits) should be received before frame lock is deemed to have been achieved. The decoder should be designed such that frame lock will be re-established within 34 000 bits after an error corrector framing phase change.

Note — This assumes that the video data does not contain three correctly phased emulations of the error correction framing sequence during the relocking period.

ANNEX A

(to Recommendation H.261)

Inverse transform accuracy specification

A.1	Generate random integer pel data values in the range -L to +H according to the random number generator given below (“C” version). Arrange into 8 by 8 blocks. Data set of 10 000 blocks should each be generated for (L = 256, H = 255), (L = H = 5) and (L = H = 300).

A.2	For each 8 by 8 block, perform a separable, orthonormal, matrix multiply, forward discrete cosine transform using at least 64-bit floating point accuracy.

µ

§

with u, v, x, y = 0, 1, 2, . . .,7

where	x,y 	= spatial coordinates in the pel domain,

u,v 	= coordinates in the transform domain,

C(u) =  for u = 0, otherwise 1,

C(v) =  for v = 0, otherwise 1.

A.3	For each block, round the 64 resulting transformed coefficients to the nearest integer values. Then clip them to the range -2048 to +2047. This is the 12-bit input data to the inverse transform.

A.4	For each 8 by 8 block of 12-bit data produced by § A.3, perform a separable, orthonormal, matrix multiply, inverse discrete transform (IDCT) using at least 64-bit floating point accuracy. Round the resulting pels to the nearest integer and clip to the range -256 to +255. These blocks of 8 ´ 8 pels are the reference IDCT input data.

A.5	For each 8 by 8 block produced by § A.3, apply the IDCT under test and clip the output to the range -256 to +255. These blocks of 8 ´ 8 pels are the test IDCT output data.

A.6	For each of the 64 IDCT output pels, and for each of the 10,000 block data sets generated above, measure the peak, mean and mean square error between the reference and the test data.

A.7	For any pel, the peak error should not exceed 1 in magnitude.

For any pel, the mean square error should not exceed 0.06.

Overall, the mean square error should not exceed 0.02.

For any pel, the mean error should not exceed 0.015 in magnitude.

Overall, the mean error should not exceed 0.0015 in magnitude.

A.8	All zeros in must produce all zeros out.

A.9	Re-run the measurements using exactly the same data values of step 1, but change the sign on each pel.

"C" program for random number generation

/* L and H must be long, that is 32 bits */
	long rand		(L,H)
	long			 L,H;
	{

static long randx = 1;	/* long is 32 bits */
		static double z = (double) 0x7fffffff;

long i,j;
		double x;				/* double is 64 bits */

randx = (randx * 1103515245) + 12345;
		i = randx & 0x7ffffffe;	   /* keep 30 bits */
		x = ( (double)i ) /  z;		   /* range 0 to 0.99999 ... */
		x * = (L+H+1);		   /* range 0 to < L+H+1 */
		j = x;				   /* truncate to integer */
		return( j - L);		   /* range -L to H */
	}

ANNEX B

(to Recommendation H.261)

Hypothetical reference decoder

The Hypothetical reference decoder (HRD) is defined as follows:

B.1	The HRD and the encoder have the same clock frequency as well as the same CIF rate, and are operated synchronously.

B.2	The HRD receiving buffer size is (B + 256 · K bits). The value of B is defined as follows:

B = 4Rmax/29.97 where Rmax is the maximum video bit rate to be used in the connection.

B.3	The HRD buffer is initially empty.

B.4	The HRD buffer is examined at CIF intervals . If at least one complete coded picture is in the buffer then all the data for the earliest picture is instantaneously removed (e.g. at tn+1 in Figure B-1/H.261). Immediately after removing the above data the buffer occupancy must be less than B. This is a requirement on the coder output bitstream including coded picture data and MBA stuffing but not error correction framing bits, fill indicator (Fi), fill bits or error correction parity information described in § 5.4.

FIGURE B-1/H.261

To meet this requirement the number of bits for the (n+1)th coded picture dn+1 must satisfy:

µ

§

where:

bn is buffer occupancy just after the time tn,

tn is the time the nth coded picture is removed from the HRD buffer,

R(t) is the video bit rate at the time t.

ANNEX C

(to Recommendation H.261)

Codec delay measurement method

The video encoder and video decoder delays will vary depending on implementation. The delay will also depend on the picture format (QCIF, CIF) and data rate in use. This annex specifies the method by which the delay figures are established for a particular design. To allow correct audio delay compensation the overall video delay needs to be established from a user perception point of view under typical viewing conditions.

FIGURE C.1/H.261

Point A is the video input to the video coder. Point B is the channel output from the video terminal (i.e. including any FEC, channel framing, etc.). Point C is the video output from the decoder.

A video sequence lasting more than 100 seconds is connected to the video coder input (point A) in Figure C-1/H.261 above. The video sequence should have the following characteristics:

—	it should contain a typical moving scene consistent with the intended purpose of the video codec;

—	it should produce a minimum coded picture rate of 7.5 Hz at the bit rate in use;

—	it should contain a visible identification mark at intervals throughout the length of the sequence. The visible identification should change every 97 video input frames and be located within the picture area represented by the first GOB in the picture. For example, the first block in the picture could change from black to white at intervals of 97 video frame periods. The identification mark should be chosen so that it can be detected at point B and does not significantly contribute to the overall coding performance.

The codec and video sequence should be arranged so that the bitstream contains less than 10% stuffing (MBA stuffing + error correction fill bits).

The encoder delay is obtained by measuring the time from when the visible identification changes at point A to the time that the change is detected at point B. Similarly, the decoder delay is obtained by taking measurements at points B and C.

Several measurements should be made during the sequence length and the average period obtained. Several tests should be made to ensure that a consistent average figure can be obtained for both encoder and decoder delay times.

Average results should be obtained for each combination of picture format and bit rate within the capability of the particular codec design.

Note — Due to pre- and post-temporal processing it may be necessary to take a mid-level for establishing the transition of the identification mark at points B and C.

1) 	BCH = Bose, Chaudhuri and Hocquenham (code).