VIDEO.CONF : configuration file for video devices

The file "/etc/video.conf" should exist on a workstation in order to

run the videodisc controllers. This file contains a table describing

the configuration of any and all video devices connected to a workstation.

There are four kinds of devices: SWTR (an audio/video switcher), RPD (a

record/playback device), an INPUT (a signal source which cannot be searched;

like, cable TV), and an OUTPUT (a signal sink, generally the user's display).

The routines in '/mit/visual/tools/lib/librpd.a' read in this table of

devices, and return to the user a sub-table of devices which can be controlled

by him/her based on which display (:0 or :1) he/she is connected to. Generally,

this is determined by the connections to and from the SWTR device(s). The

special case where there is no SWTR is described below.

Each entry has the following info: ( {}'s are 1-of sets, <>'s are variables )

Type:		{ SWTR | RPD | INPUT | OUTPUT }

SysName:	if switcher: SWTR-<unit_number>; else, anything

Model:		if switcher: <model_name>_<#inputs>x<#outputs>; else, anything

TtyPort:	name of serial port in /dev that device connects to, if any

Baud:		(baud rate, if any): { 9600 | 4800 | 2400 | 1200 | 600 | 300 }

Parity:		if any: { none | any | even | odd }

Chan:		<switcher#>-<channel#>; Type (above) determines whether

this is an input or output channel

1. a switcher entry must come before any devices that connect to it

2. a switcher's 'switcher' designation in the channel entry must

match its unit number contained in the SysName

3. a switcher's 'channel' designation in the channel entry must be zero ('0')

4. all sources/sinks must connect to a valid switcher in the table

5. sources/sinks can specify the special switcher unit '0' if disconnected,

or if no switcher exists

6. each sysname, ttyport, and channel in the table must be unique

7. nonsensical entries for INPUT/OUTPUT channels should be marked '?'

8. the SysName of an OUTPUT channel refers to the destination

of that channel; so 'E40-358C-2:0' would mean that the channel connects

to a workstation; or 'CH-10' would mean that it connects to campus

cable channel 10. The special names 'unix:0' and 'unix:1' are converted

to the fully-expanded display variable appropriate for the user on

either 'hostname':0 or 'hostname':1, respectively. If an output's name

is a '>' followed by the name of another device, that device must be

an RPD, and is assumed to be capable of recording. Thus valid names

for output devices are as follows:

a workstation display:	<hostname>:<display_num>

a cable TV channel:	CH-<channel_num>

an RPD (recorder):	>RPD_device_name

10. the '#' character begins a comment line; alternative is a whitespace

character at the start of the line

11. if a configuration lacks a switcher, the software will find that the

OUTPUT device's channel for the user's display is set to either 0-1

(if on ttyv0) or 0-2 (if on ttyv1). There will be exactly one matching

device whose channel spec is the same (0-1 or 0-2). The table

returned to the user will contain the OUTPUT and one RPD (if it exists).

The following is an example of a fully-loaded 2-headed system, with two disc

players, a cable TV input, a video server input, and a switcher for each

display.

#

# Type	SysName		Model		TtyPort	Baud	Parity	Chan

# ===================================================================

#

# this is for the display `hostname`:0

#

SWTR	SWTR-1		ATHENA_4x1	tty02	1200	none	1-0

RPD	DISC-1		DEC_VDP50	tty04	9600	none	1-1

INPUT	TV-1		?		?	?	?	1-2

INPUT	SERV-1		?		?	?	?	1-3

RPD	DISC-3		DEC_VDP50	tty06	9600	none	1-4

OUTPUT	unix:0		?		?	?	?	1-1

#

# this is for the display `hostname`:1

#

SWTR	SWTR-2		ATHENA_4x1	tty03	1200	none	2-0

RPD	DISC-2		DEC_VDP50	tty05	9600	none	2-1

INPUT	TV-2		?		?	?	?	2-2

INPUT	SERV-2		?		?	?	?	2-3

RPD	DISC-4		DEC_VDP50	tty07	9600	none	2-4

OUTPUT	unix:1		?		?	?	?	2-1

#

#

#

# --------------------------------------------------------------------------

#

# following is an example of a 'video.conf' file for a video server machine

#

# Type	SysName		Model		TtyPort	Baud	Parity	Chan

# ==================================================================

# first, the routing switcher; say, a 10 inputs by 6 output matrix

SWTR	SWTR-3		3M_10x6		tty08	9600	none	3-0

# define all of the inputs to the switcher

RPD	RDISC-1		DEC_VDP50	tty09	9600	none	3-1

RPD	RDISC-2		DEC_VDP50	tty10	9600	none	3-2

RPD	RDISC-3		DEC_VDP50	tty11	9600	none	3-3

RPD	RDISC-4		DEC_VDP50	tty12	9600	none	3-4

RPD	RDISC-5		DEC_VDP50	tty13	9600	none	3-5

RPD	RDISC-6		DEC_VDP50	tty14	9600	none	3-6

INPUT	ABC		?		?	?	?	3-7

INPUT	NBC		?		?	?	?	3-8

INPUT	CBS		?		?	?	?	3-9

#

# now define all of the outputs from the switcher

#

OUTPUT	11-132-1:0	?		?	?	?	3-1

OUTPUT	11-132-1:1	?		?	?	?	3-2

OUTPUT	11-132-2:0	?		?	?	?	3-3

OUTPUT	11-132-2:1	?		?	?	?	3-4

OUTPUT	11-132-3:0	?		?	?	?	3-5

OUTPUT	11-132-3:1	?		?	?	?	3-6

OUTPUT	CH-10		?		?	?	?	3-7

#

#

# --------------------------------------------------------------------------

The following is an example of a bare-minimum system, with one disc player

per display and NO switchers.

# Type	SysName		Model		TtyPort	Baud	Parity	Chan

# ==================================================================

# this is for the display `hostname`:0

RPD	DISC-1		DEC_VDP50	tty00	9600	none	1-1

OUTPUT	unix:0		?		?	?	?	1-1

# this is for the display `hostname`:1

RPD	DISC-2		DEC_VDP50	tty01	9600	none	2-1

OUTPUT	unix:1		?		?	?	?	2-1

#

# end of examples

#

# --------------------------------------------------------------------------

More Detailed Stuff:

1. switcher channels (esp. on 2-headed machine)

the switcher channel entries in the table are of the form:

<switcher>-<channel_number>

where <switcher> represents the number of the switcher (SWTR-#)

that the channel refers to. So to refer to channel 3 of switcher

1 (that's SWTR-1), the entry would be "1-3". In the two-headed

situation, the output of SWTR-1 would probably be connected to

the display <hostname>:0 and SWTR-2 would be connected to the

display <hostname>:1.

2. input/output format:

INPUT	refers to an audio/video input ONLY; no associated device.

TV refers to a campus cable input from a local selector. Other

names like BARS or CBS are undefined, but assumed to be direct

channel connects from a remote selector.

OUTPUT	refers to the thing that a switcher output channel connect to.

<host>:<display#> refers to a video workstation.

CH-<number> might refer to a channel of campus cable

that this server feeds.