JIS@BITSY.MIT.EDU              June 9, 1987                         *DRAFT*

Jeffrey I. Schiller

A Protocol for the Dynamic

Assignment of IP Addresses

for use on an Ethernet

Abstract

This  document  describes  a  simple protocol to perform dynamic IP

address assignment on an Ethernet network. The protocol is adaptable to  be

useful  on  any  networking technology that supports broadcast or multicast

and uses the ARP protocol (ARP is described in RFC826).

Problem

The number of single user workstations that use the IP/TCP protocol

suite  is  growing.  More  importantly more of these workstations are being

installed by "end users", people who  are  not  networking  experts.    One

problem  that results from this is that not all workstations that are setup

by non-experts get correctly configured to  use  the  network.    This  can

create   problems   both   for   the  user  of  the  improperly  configured

workstations, and more importantly it can create problems for  other  users

on  the network. The two typical parameters that have to be configured into

a workstation are its own IP address and the  IP  address  of  the  default

gateway.

The   assignment  of  the  workstations  IP  address  presents  the

thorniest problem because the manual that comes with the workstation cannot

tell  you  your  IP  address.  It  is  necessary for the user to contact an

appropriate (and typically  different  for  each  Ethernet)  administrative

person in order to get a uniquely assigned IP address.

Requirements

A  protocol  to  dynamically assign IP addresses must be capable of

operating on a network that also has on it other workstations/hosts that do

not  participate  in the address assignment protocol, without the addresses

in use by those hosts being usurped.

The protocol should operate as independently from human interaction

as  possible.  In  otherwords  a  user  of a workstation should not have to

customize his workstation in any way. If any network configuration commands

are  used  at all they should be of the form "network on" and "network off"

JIS@BITSY.MIT.EDU              June 9, 1987                         *DRAFT*

Jeffrey I. Schiller

with the protocol handling all other details.

Therefore  the  protocol  must  be  capable  of  transmitting   the

following   information   to  the  workstation  (during  a  time  when  the

workstation does not yet have an IP address):

The network number of the Ethernet.

The address of the default gateway.

The list of addresses that are permanently assigned to  hosts  that

do not participate in the protocol.

The  protocol should not rely on a sole information providing host,

as this introduces a single point of failure.

Proposed protocol

This protocol operates at the Ethernet  (or  802.3)  level  of  the

network.   There are two pieces, the first piece can be called the "network

information protocol" and is responsible for providing the workstation with

the  information  it  will  need  to  configure  itself.    Armed with this

information the workstation can then make a first stab at issuing itself an

IP  address.  The  second  part of the protocol involves the use of the ARP

protocol. The workstation attempts to use the ARP protocol  to  resolve  an

Ethernet  address  for  the IP address it intends to use. If after N [N not

defined yet] resends of the ARP resolve request  it  does  not  receive  an

answer,  then it assumes that the IP address it picked is available for use

and configures that address as its IP address.

It is important that the workstation pick an IP address that is not

1

likely to be picked by another workstation.   To facilitate this there  are

two ways that a workstation should use to decide its first "guess" at an IP

address.   The  Network  Information  Protocol  may  provide  the  invoking

workstation with a first guess as to what its IP address should be (more on

this later).  If this information is provided the  workstation  should  use

this  as a first guess. If this information is not provided (ie. that field

in the packet is zero), then the host should construct its own first  guess

using  the  Network address (and network mask) and constructing a likely to

be unique host portion.  A hash function that is fed the  Ethernet  address

of the workstation as a seed should suffice for this. It is defined below:

_______________

1

"Important"  in  as much as it is felt that a workstation should tend to

be assigned the same IP address if at all possible across reboots.

JIS@BITSY.MIT.EDU              June 9, 1987                         *DRAFT*

Jeffrey I. Schiller

[exact hash function should be specified here, but isn't yet]

It  is  important  to  understand that this arrives only at a first

guess of what IP address the workstation should use.  If  that  address  is

determined  to  be  in use by using ARP as described above, the workstation

should construct a second attempt and try again. The hash  function  should

be modified as follows:

[modification to nonexistent hash function should go here]

Of  course  once  it  is using an IP address, if other workstations

attempt to assign that address, normal ARP replies will act to "defend" its

address.

Network Information Protocol

The  network  information  protocol  will  operate  directly on the

Ethernet layer. It will use a Ethernet type value that is unique to it  (it

is  not  implemented  as  an  extension  of  the  ARP  protocol, as correct

functioning with all existing ARP implementations cannot be verified).

To start the protocol a workstation sends a broadcast packet  which

requests the information.

Upon  receiving  such  a request, all workstations will forward the

information to the requesting workstation (hopefully  this  information  is

relatively  constant).  To  prevent  too many collisions and sudden network

congestion, each workstation will wait  a  random  amount  of  time  before

responding.  One  workstation/host on the network will NOT wait, so that in

general it will  be  the  primary  information  provider.  In  general  the

"primary"  provider  will be the default gateway (though it doesn't have to

be). The "primary" provider will also be the only information provider that

will fill in the otherwise optional "Recommended IP address" field.  It can

obtain this information by using a "Reverse ARP" lookup on its  ARP  cache.

The  word  "can"  should  be emphasized here, although reverse ARP may be a

clever way to help pick an IP address, it is not the only way that this can

be  done.  This protocol specification does not preclude the use of another

method.

[Reference should be made to the originator of  the  "Reverse  ARP"

idea, it is not new here!]

A  workstation which has not yet determined its IP address WILL NOT

respond to an information request.

The  information  response  packet  will  contain   the   following

information.

A  Version  number,  because version numbers a good and virtuous in

that they allow future enhancement of the protocol in an upward  compatible

JIS@BITSY.MIT.EDU              June 9, 1987                         *DRAFT*

Jeffrey I. Schiller

way!

Network  address  of the network. This will be an IP address with a

host portion field of 0.

A network mask. This will a 32bit quantity with  ones  masking  off

the network number portion of the IP address.

A  broadcast  address.  The  broadcast  address  of  choice on this

network.

The lowest available IP address for dynamic assignment.

The highest available IP address for dynamic assignment.

+--------------------------------+

|Source Ethernet Address         |

+----------------+---------------+

|Source Ethernet |Checksum       |

+----------------+---------------+

|Opcode          |Version        | <-- Version = 1

+----------------+---------------+ <-- Below here opcode dependent

|Network Address                 |     These fields are only used

+--------------------------------+     when opcode = Response

|Network Mask                    |

+--------------------------------+

|Broadcast Address               |

+--------------------------------+

|Lowest usable IP address        |

+--------------------------------+

|Highest usable IP address       |

+--------------------------------+

|Recommended IP address          | <-- Either a first guess IP address

+--------------------------------+     to use, or zero

Checksum = Ones compliment of the ones compliment checksum of the packet.

(Ie. same algorithm as IP)

Opcode   = 1 Information request.

2 Information response.

Commentary

One of the important issues that an implementor  of  this  protocol

needs to be aware of, is how the "system" will react to sudden interruption

of all workstations on the network (ie. a  power  failure).  Things  should

initialize  correctly  even  if  all  devices  on  the network are suddenly

reinitializing. In other words someone HAS  to  be  the  authority  on  the

JIS@BITSY.MIT.EDU              June 9, 1987                         *DRAFT*

Jeffrey I. Schiller

information  provided  in the "Network Information Protocol." The authority

presumably KNOWS what this information is and doesn't have to ask  when  he

initializes.

Consider the following scenario:

A network can be setup so that the default gateway is the "primary"

(and authoritative) information provider. We assume that the gateway  KNOWS

its  configuration  through  mechanisms  outside  of this protocol and thus

KNOWS its IP address on the ethernet and the other information provided  by

the "Network Information Protocol."

If  all systems reboot simultaneously (as after a power outage) the

following occurs: Workstations will broadcast "Network Information Request"

packets.  Until the gateway comes up, noone will respond (because until you

know your IP address you must not respond to Network Information Requests).

When the gateway comes up then he can start responding and the workstations

come up. If Reverse ARP is normally used by  the  gateway  to  provide  the

"Recommended  IP  address",  this  information  can  be  left  out (because

obviously the gateway's ARP cache is probably empty at this point).

Of course in this scenario the workstations will fail to initialize

their  network  interfaces  unless (or until) the gateway comes up. This is

probably a reasonable price to pay, for chances  are  that  candidates  for

this  protocol  probably  depend on other network services provided through

that gateway (like RFC882 domain name resolution for  example)  and  cannot

get  much  utility  out  of  the  network until the gateway comes up anyway

[hmmm.].