The front panel emulator for the now machine is based on micro-ODT for

the LSI-11.  In the command descriptions:

1) Case is ignored.

2) Addresses and other numerical values are entered in hex.

Hex numbers may be preceeded by '0x' or '0X' to distinguish

them from register names.

3) Register names follow the standard 68000 conventions:

D0-D7, A0-A6, SP, USP, PC, SR.

4) Delete (DEL) may be used to erase characters; ^U may be used

to cancel an entire line.

5) Square brackets ("[" "]") delimit optional arguments.

The following commands are available:

?		Display a short help screen.

[<address>],	Open the specified address as a byte and display its

contents in hex.  If no address is specified, open the

current address.

In the ',', ';', and '/' commands, an exclamation mark

('!') may be used as a separator between the address (if

any) and the command; this results in the location being

opened but not read and no value is displayed.  This is

occasionally useful when the address being opened is the

address of a device register in I/O space and reading the

device register affects the device in some way.

[<address>];	Open the specified address as a word and display its

contents in hex.  If no address is specified, open the

current address.

[<address>]/	Open the specified address as a longword and display its

contents in hex.  If no address is specified, open the

current address.

[<address>]\	Open the specified address as a byte and display its

contents as an ASCII character.  If no address is specified,

open the current address.

[<value>]<CR>	Deposit the specified value (if any) in the currently open

location and close it.

[<value>]<LF>	Deposit the specified value (if any) in the currently open

location, close it, and open the next location.

[<value>]^	Deposit the specified value (if any) in the currently open

location, close it, and open the previous location.

[<value>]@	Deposit the specified value (if any) in the currently open

location, close it, and open the location pointed to by it.

[<address>]G	Go.  Begin execution of the program residing at the specified

address, or at the current address if none is specified.  This

command causes a processor RESET instruction to be executed

before control passes to the user program.  Execution begins

at processor priority 7 in supervisor mode.

P		Proceed.  Continue execution of the user program at the

address currently in the PC.  If the ODT was entered as

a result of a trap or direct call (as opposed to external

reset or power-up) the appropriate RTE or RTS will be

performed.

I		Initialize.  This simulates a power-on or external reset by

executing a RESET instruction and then jumping to the

power-on code.

L		Load a file from the download serial line and begin executing

it at its specified load address.  Format of the boot file

is to be specified later.

N		Network bootload.  This command may take an optional argument

specifying the server to boot from and/or the remote disk

or file to boot from.

T		Terminal slave.  Connect the terminal port to the download

port.  Continues until the sequence "^^q" (that is, control-

uparrow q) is received on either line.  To send a control-

uparrow it must be typed twice.

Operation of the front panel emulator is as follows.  Control can enter the

emulator in three ways:

1) By a power-on or external reset, forcing the processor to trap

through address zero.

2) By a trap or interrupt through an otherwise uninitialized vector.

The power-on code initially sets all vectors to trap to the

emulator.

3) By a direct call to the emulator's external entry point.

Operation of the program for each of the three entry points is described

below.

1) Power-On Entry

Control enters the power-on entry when a power-up or external reset occurs,

and on execution of the Initialize command.  The code at power-up performs

some machine initialization and validity checks before entering the command

loop.  It begins by initializing the serial I/O card and outputting a

carriage return to both the download port and the direct terminal port.

Next, it performs a quick check of the system RAM , then sets up its stack

and initializes its state information, which is stored at the base of its

stack.  If unsuccessful in setting up the stack (RAM check fails), it

displays an error code on both ports and halts.  (Error codes are listed

at the end).  It next proceeds to checksum the emulator ROM; again, if the

checksum fails an error code is displayed on both ports and the emulator halts.

Next, the trap vectors are set up to point to the emulator's trap entry.

Finally, the main command loop is called and the prompt ('.') is displayed.

The main loop polls both ports for commands and outputs to both ports.

If the main loop returns, the registers are loaded from the state vector

and control is passed to the user program at the address stored in the

user PC.

2) Trap Entry

If an unexpected trap or exception occurs through an otherwise uninitialized

trap or interrupt vector, control passes to the emulator.  The emulator

disables further interrupts, then attempts to set up its state vector at the

top of the current supervisor stack (this includes saving the user's registers

in the state vector).  The current state of the serial ports is saved on

the stack and the serial ports are set up as needed by the emulator (the

state will be restored on proceed).  Finally, the main command loop is

called and the prompt displayed.  If the main loop ever returns, the serial

port state is restored, the user registers are restored, and a Return from

Exception is performed to return control to the user program.

3) Explicit Call

The front panel emulator provides an entry point which may be called from the

user program, eg. when the user program bughalts.  It performs the same

functions as the trap entry point.  If the main loop ever returns, the

serial port state and user registers are restored, and a Return from

Subroutine is performed to return control to the user program.

The following error codes are defined:

1		RAM error.  Main RAM could not be found or the simple RAM

diagnostics run at power-up time failed.

2		ROM checksum error.  The ROM checksum did not match, and hence

the ROM is probably corrupted.

3		Address error.  The user attempted to read or modify a

non-existent address.

4		Trap entry.  The user program took an exception which vectored

to the emulator.

5		Emulator call.  The user program explicitly called the

emulator.