|  |  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- | --- |
|  |  | ITU-T | G.983.8 |  |
|  |  | TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | (03/2003) |
|  | SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS Digital sections and digital line system – Optical line systems for local and access networks |  |  |  |
|  | B-PON OMCI support for IP, ISDN, Video, VLAN Tagging, VC Cross-Connections and other select functions |  |  |  |
|  | CAUTION ! PREPUBLISHED RECOMMENDATION This prepublication is an unedited version of a recently approved Recommendation. It will be replaced by the published version after editing. Therefore, there will be differences between this prepublication and the published version. |  |  |  |

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU [had/had not] received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2003

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without the prior written permission of ITU.

ITU-T Recommendation G.983.8

B-PON OMCI support for IP, ISDN, Video, VLAN Tagging, VC Cross-Connections and other select functions

Summary

This Recommendation provides ONT Management and Control Interface (OMCI) support for the Broadband Passive Optical Network (B-PON) system defined in ITU-T G.983.1 [1] for select functions which were out of the scope of ITU-T G.983.2 [2].  OMCI support is specified for Internet Protocol (IP) Router functionality on Local Area Network (LAN) cards, Integrated Services Digital Network (ISDN) interfaces, additional Ethernet performance monitoring, video interfaces, virtual LAN (VLAN) tagging, extended Media Access Control (MAC) Bridge Filtering, local craft terminal interfaces, virtual channel (VC) cross-connections, and Optical Network Units (ONUs).

# Scope

This Recommendation focuses on the OMCI specifications related to support for IP Router functionality on LAN cards, ISDN, additional Ethernet performance monitoring, video interfaces, VLAN tagging, extended MAC Bridge Filtering, local craft terminal interfaces, VC cross-connections, and ONUs (detailed management for ONUs with xDSL interfaces is for future study). Though the OMCI specifications are based on ITU-T G.983.2 [2], some enhancements are needed. The scope of this Recommendation is limited to the enhancements only.

This Recommendation includes additions and editorial changes to sections of ITU-T G.983.2 [2] that pertain to these topics.  As this Recommendation serves as an extension of ITU-T G.983.2 [2], all sections of that Recommendation remain pertinent.

# References

The following ITU-T Recommendations and other references contain provisions that, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

[1]	ITU-T G.983.1 (1998), Broadband optical access systems based on Passive Optical Networks (PON).

[2]	ITU-T G.983.2 (2002), ONT management and control interface specification for ATM PON.

[3]	IETF RFC 2096 (1997), IP Forwarding Table MIB.

[4]	IEEE 802.1Q (1998), Virtual Bridged Local Area Networks.

[5]	IEEE 802.1D (1998), Media Access Control (MAC) Bridges.

# Abbreviations

This Recommendation uses the following abbreviations:

ANI	Access Node Interface

APON	ATM over Passive Optical Network

ARP	Address Resolution Protocol

B-PON	Broadband Passive Optical Network

CFI	Canonical Format Indicator

- CNR			Carrier-to-Noise Ratio

- CSO			Composite Second Order

- CTB			Composite Triple Beat

ICMP	Internet Control Message Protocol

IP	Internet Protocol

ISDN	Integrated Services Digital Network

LAN	Local Area Network

ME	Managed Entity

MIB	Management Information Base

OLT	Optical Line Terminal

OMCC	ONT Management and Control Channel

OMCI	ONT Management and Control Interface

ONT	Optical Network Terminal

PON	Passive Optical Network

TCA	Threshold Crossing Alert

TCI	Tag Control Information

TPID	Tag Protocol Identifier

UNI	User Network Interface

VCC	Virtual Channel Connection

VID			VLAN Identifier

VLAN	Virtual Local Area Network

VP	Virtual Path

# Reference Model and Terms

## OMCI in ITU-T Recommendation G.983.omci.newservices

See clause 4.1/G.983.2.

## ONT Functions

See clause 4.2/G.983.2.

## VP Mux functionality in the ONT

See clause 4.3/G.983.2.

# Requirements of the management interface specification

See clause 5/G.983.2.

## Configuration management

Modify the text in clause 5.1/G.983.2 to allow either a VP or VC cross-connection to be used in the ONT.

Delete the following text.

“ATM VC management is not a part of this Recommendation (see [App. V-1] and [9] in clause 2).   VC cross-connection capability is not necessary for the ONT, as VC cross-connection is handled by the OLT.  Note that”

Change figure caption for Figure 2-1/G.983.2 to “VP cross-connection termination model”.

Add the following text and figure at the end of clause 5.1/G.983.2.

“For cases such as the support of ONUs with xDSL interfaces, it is necessary to implement VC cross-connections.  VC cross-connection can be supported by the ONT/ONU, as indicated by the termination model shown in Figure 1/G.983.8.

OLT

ONT

ANI

(B-PON)

NNI

VP-XC

VC-XC

UNI

(not ATM)

ANI

(B-PON)

VP-XC

VC

VP

VP

VC

VP

VP

VC

VC

VC

VC-XC

Figure 1/G.983.8 – VC cross-connection termination model

## Fault management

See clause 5.2/G.983.2.

## Performance management

The following performance management-related managed entities are added to the list given in clause 5.3/G.983.2.

m)	IP Router PM History Data1

n)	IP Router PM History Data2

o)	ICMP PM History Data1

p)	ICMP PM History Data2

q)	VC PM History Data

r)	Ethernet Performance Monitoring History Data2

## Security management

See clause 5.4/G.983.2.

# Protocol-independent MIB for the OMCI

See clause 6/G.983.2.

## Managed entities associated with IP Router function on LAN card, ISDN, VLAN tagging, extended MAC Bridge Filtering, video card interfaces, local craft terminal interfaces, VC cross-connections, additional Ethernet performance monitoring, and ONUs

The managed entities in Table 1/G.983.8 are defined in addition to the managed entities defined in Recommendation G.983.2.

The definitions of the managed entities in Table 2/G.983.8 are modified from those given in Recommendation G.983.2.

Table 1/G.983.8	Additional managed entities in the OMCI

| Managed Entity | Required/Optional | Description |
| --- | --- | --- |
| IP Port Configuration Data | CR | Used for IP Port supported by the ONT |
| IP Router Service Profile | CR | Used for IP Router supported by the ONT |
| IP Router Configuration Data | CR | Used for IP Router supported by the ONT |
| IP Router PM History Data1 | O | Used for IP Router performance monitoring |
| IP Router PM History Data2 | O | Used for IP Router performance monitoring |
| ICMP PM History Data1 | O | Used for ICMP performance monitoring |
| ICMP PM History Data2 | O | Used for ICMP performance monitoring |
| IP Route Table | CR | Used for IP Router supported by the ONT |
| IP Static Routes | CR | Used for IP Router supported by the ONT |
| ARP Service Profile | CR | Used for IP Port supported by the ONT |
| ARP Configuration Data | CR | Used for IP Port supported by the ONT |
| VLAN Tagging Operation Configuration Data | O | Used for VLAN tagging |
| MAC Bridge Port Filter Preassign Table | O | Used for Ether type filtering |
| Physical Path Termination Point ISDN UNI | O | Used for ISDN port supported by the ONT |
| Physical Path Termination Point Video UNI | O | Used for video port |
| Physical Path Termination Point Video ANI | O | Used for video input port |
| Physical Path Termination Point LCT UNI | O | Used for local craft terminal port |
| VLAN Tagging Filter Data | O | Used for VLAN tagging |
| ONUB-PON | O | Used for ONU that supports xDSL interfaces |
| ATM VC Cross-Connection | O | Used for VC multiplexing with VCI translation in the ONU |
| VC Network CTPB-PON | O | Used for VC link termination in the VC MUX |
| VC PM History Data | O | Used for VC performance monitoring |
| Ethernet Performance Monitoring History Data 2 | O | Used for Ethernet performance monitoring |

Table 2/G.983.8	Modified managed entities in the OMCI

| Managed Entity | Required/Optional | Description |
| --- | --- | --- |
| ONTB-PON | R | Used for ONT equipment management |
| ONT Data | R | Used for OMCI MIB management |
| Subscriber Line Card | CR* | Used for the UNI line card plug-in |
| TC AdapterB-PON | CR | Used for TC layer at the UNI side for the ATM UNI |
| Voice Service Profile AAL | CR | Used for Voice over AAL1/2 supported by the ONT |
| Voice CTP | CR | Used for Voice supported by the ONT |
| Voice PM History Data | O | Used for Voice performance monitoring |
| Interworking VCC Termination Point | CR | Used for non-ATM UNIs |
| UPC Disagreement Monitoring History DataB-PON | CR | Used for the ONT that supports UPC |
| Threshold DataB-PON | CR | Used for the set-up of threshold values |

* 	Note: the preferred solution is that the Subscriber Line Card and Subscriber Line Cardholder managed entities should always be modelled, regardless of whether or not the ONT has integrated interfaces; however, for reasons of backward compatibility, these managed entities remain as “CR.”

## Managed Entity relation diagrams

### New Managed Entity relation diagrams

Figure 2/G.983.8 shows the managed entity relation diagram for IP Router service in an ONT that models VP cross-connection functionality.

Figure 2/G.983.8 - Managed Entity relation diagram, IP Router service in an ONT that models VP cross-connect functionality

The "*" in Figure 2/G.983.8 indicates that the Traffic Descriptor can be one of the specific Traffic Descriptor managed entities defined in clause 7.5.2/G.983.2.

Figure 3/G.983.8 shows the managed entity relation diagram for an ONT that supports MAC bridge service with preassigned MAC addresses.

Figure 3/G.983.8  Managed Entity relation diagram, MAC bridge service including VLAN services in an ONT that models VP Cross-connect functionality

The "*" in Figure 3/G.983.8 indicates that the Traffic Descriptor can be one of the specific Traffic Descriptor managed entities defined in clause 7.5.2/G.983.2.

In addition, the managed entity relation diagrams for ONTs that support VC cross-connections can be derived from Figures 4, 5, 7, 9-a, 9-b, and 9-c/G.983.2 by replacing “VP Network CTPB-PON” with “VC Network CTPB-PON” and “ATM VP Cross-Connection” with “ATM VC Cross-Connection”.

### Revised managed Entity relation diagrams

Replace Figure 9-b/G.983.2 with Figure 4/G.983.8.  Modifications fromFigure 9-b/G.983.2 are indicated by underlined text.

Figure 4/G.983.8  Managed Entity relation diagram, Voice over AAL2 service in an ONT that models VP cross-connect function

# MIB Description

Providing OMCI support for IP Router function, ISDN, additional Ethernet performance monitoring, video interfaces, VLAN tagging, extended MAC Bridge Filtering, local craft terminal interfaces, VC cross-connections, and ONUs causes the modification of existing managed entities and creation of new managed entities. In this clause, only modified or new managed entities are described. All other managed entities in Recommendation G.983.2 [2] remain unchanged.

## Modified managed entities

Where appropriate, modifications from the managed entity descriptions in G.983.2 [2] are indicated by underlined or crossed-out text.

### ONTB-PON

Modify the “VP cross-connection function option” in clause 7.1.1/G.983.2 as indicated below to allow support of VC cross-connections.

| VP/VC cross-connection function option: | This attribute identifies the support of ATM VP or VC cross-connection management functions for the interworking connections to non-ATM UNIs. The value is set to 0x00 if ATM VP or VC cross-connection management functions are not modelled. The value is set to 0x01 if the ATM VP cross-connection management functions are modelled. The value is set to 0x02 if ATM VC cross-connection management functions are modelled. The default value of this attribute is 0x01. (R) (mandatory) (1 byte) |
| --- | --- |

### ONT Data

To support ONUs, all references to ONT should be replaced by ONT/ONU in clause 7.1.2/G.983.2.

### Subscriber Line Cardholder

Add the following entry to the list of line card types given in Table 3/G.983.2.

| Coding | Contents | Description |
| --- | --- | --- |
| 39 | LCT | Local craft terminal interface |

### Subscriber Line Card

Add the following attribute to the end of the list of attributes given in clause 7.1.4/G.983.2:

| CardConfiguration: | This attribute is used to select the appropriate configuration on configurable line cards (e.g., T1/E1). Table 3/G.983.2 specifies 3 configurable card types: A45/34 (code 9), C-DS1/E1 (code 16), and C-DS1/E1/J1 (code 17). Values are indicated below for the allowed card types and configurations. Card Type Configuration Value A45/34 ATM 44.736 Mbps 0x00 ATM 34.368 Mbps 0x01 C-DS1/E1 DS1 0x00 E1 0x01 C-DS1/E1/J1 DS1 0x00 E1 0x01 J1 0x02 (R, W, Set-by-create (if applicable)) (mandatory for configurable line cards) (1 byte) |
| --- | --- |

### Physical Path Termination Point Ethernet UNI

Add the attribute shown below to the end of the attribute list for the Physical Path Termination Point Ethernet UNI in clause 7.3.2/G.983.2.  This attributes allow the port to be configured to discard any packets that are not PPPoE packets. This will allow more efficient use of the PON bandwidth for carriers that offer a service that uses PPPoE.

| PPPoE Filter: | This attribute may be used to control filtering of PPPoE packets on this Ethernet port. The value 0x00 is used to disable filtering. The value 0x01 is used to enable filtering. When filtering is enabled, all packets other than PPPoE packets will be discarded. Default value is 0x00. (R,W) (optional) (1 byte) |
| --- | --- |

### Interworking VCC Termination Point

### Interworking Termination Point pointer attribute

Modify the Interworking Termination Point pointer attribute in clause 7.3.7/G.983.2 as indicated below to allow ISDN support.

| Interworking Termination Point pointer: | This attribute provides a pointer either to the associated instance(s) of the following managed entities (depending on the service provided): - Physical Path Termination Point Ethernet UNI - Physical Path Termination Point POTS UNI - Physical Path Termination Point CES UNI - Logical Nx64kbit/s sub-port Connection Termination Point - (Physical Path Termination Point ISDN UNI. Note: for the case utilizing the multiplexing function of AAL2, this attribute is assigned a special value: - 0x00XX will be used for pseudo slotIDs, - 0xXX00 will be used for pseudo portIDs. Therefore, 0x0000 will be used only for the case that the integrated interfaces (integrated type of ONT) support AAL2 multiple function. (R, Set-by-create) (mandatory) (2 bytes) |
| --- | --- |

### VPNetworkCTP Connectivity Pointer attribute

Update to allow for either VP or VC Network CTPs to be used.  Replace VPNetworkCTP Connectivity Pointer attribute in clause 7.3.7/G.983.2 with the updated attribute given below.

| VP/VCNetworkCTP Connectivity Pointer: | This attribute provides an instance identifier of the VP Network CTP or VC Network CTP that is associated with this Interworking VCC Termination Point. (R, Set-by-create) (mandatory) (2 bytes) |
| --- | --- |

### Threshold DataB-PON

The following managed entities are added to the list of thresholded managed entities given in Relationships section of clause 7.3.17/G.983.2.

•	IP Router PM History Data1

•	IP Router PM History Data2

•	ICMP PM History Data1

•	ICMP PM History Data2

•	VC PM History Data

•	Ethernet Performance Monitoring History Data 2

### Voice Service Profile AAL

Modify the definition for the Managed Entity id attribute in clause 7.3.24/G.983.2 as indicated below.

| Managed Entity id: | This attribute provides a unique number for each instance of this managed entity. This 2-byte number is directly associated with the id of the Physical Path Termination Point POTS UNI or Physical Path Termination Point ISDN UNI. (R, Set-by-create) (mandatory) (2 bytes) |
| --- | --- |

### Voice CTP

Modify clause 7.3.27/G.983.2 to allow ISDN support.

### Relationships

Modify the Relationships section in clause 7.3.27/G.983.2 as indicated below.

Relationships

Zero or more instances of this managed entity shall be contained in an instance of the ONTB-PON or Subscriber Line Card managed entity classified as a POTS or ISDN type.

### InterworkingPPTPPtr attribute

Modify the definition for the InterworkingPPTPPtr attribute in clause 7.3.27/G.983.2 as indicated below.

| InterworkingPPTPPtr: | This attribute provides a pointer to the associated instance of the Termination Point managed entity for the case of POTS or ISDN services. When this Voice CTP is associated with a Subscriber Line Card managed entity classified as a POTS type, this attribute points to the instance of the Physical Path Termination Point POTS UNI. When this Voice CTP is associated with a Subscriber Line Card managed entity classified as ISDN type, this attribute points to the instance of the Physical Path Termination Point ISDN UNI. (R, Set-by-create) (mandatory) (2 bytes) |
| --- | --- |

### Voice PM History Data

Replace the existing managed entity in clause 7.3.28/G.983.2 with the modified one given below.

Voice PM History Data

This managed entity contains the last completed 15-minute interval performance monitoring data collected as a result of monitoring a voice port on an ONT. The statistic data value is only updated at the end of each period.  Instances of this managed entity are created/deleted by the OLT after an instance of a Physical Path Termination Point POTS UNI or Physical Path Termination Point ISDN UNI managed entity is created/deleted.

Relationships

One instance of this managed entity can exist for each instance of a Physical Path Termination Point POTS UNI or Physical Path Termination Point ISDN UNI.

Attributes

| Managed Entity id: | This attribute provides a unique number for each instance of this managed entity. This 2-byte number is directly associated with the id of a Physical Path Termination Point POTS UNI or Physical Path Termination Point ISDN UNI. (R, Set-by-create) (mandatory) (2 bytes) |
| --- | --- |
| Interval End Time: | This attribute identifies the most recently finished 15-minute interval. It is a cyclic counter (modulo 0xFF (256)) that is incremented each time a new interval is finished and the attribute counters are updated. The value of this attribute is 0x00 during the first 15-minute interval that starts with the reception of the “synchronize time” action. The value is 0x01 during the first period after this, and so on. If this managed entity is created after the reception of the “synchronize time” action, the value of this attribute set equal to the number of the last completed interval. The actual counters of this managed entity start counting directly. The attribute counters are updated at the end of the interval. (R) (mandatory) (1 byte) |
| Threshold DataB-PON id: | This attribute provides a pointer to an instance of the Threshold DataB-PON managed entity that contains the threshold values for the performance monitoring data collected by this managed entity. (R, W, Set-by-Create) (mandatory) (2 bytes) |
| VoicePortBuffer Overflows: | This attribute provides the total number of payload octets dropped due to buffer overflow at this port (note: this number does not include octets that are dropped because they arrived late had already been substituted by a filler octet). If the actual counter saturates, it remains on its maximum value. (R) (mandatory) (4 bytes) |
| VoicePortBuffer Underflows: | This attribute provides the total number of filler octets injected into the active media stream on this port due to playout buffer underflow. If the actual counter saturates, it remains on its maximum value. (R) (mandatory) (4 bytes) |
| ActiveSeconds: | This attribute provides the total length of time, in seconds, that the AAL2 channel associated with this port has been active. If the counter saturates, it remains on its maximum value. (R) (mandatory) (4 bytes) |
| DchannelBuffer Overflows: | This attribute provides the total number of payload octets for the D channel that are dropped due to buffer overflow at this port (note: this number does not include octets that are dropped because they arrived late had already been substituted by a filler octet). If the actual counter saturates, it remains on its maximum value. (R) (mandatory, if this voice port carries ISDN traffic) (4 bytes) |
| B1ChannelBuffer Overflows: | This attribute provides the total number of payload octets for the B1 channel that are dropped due to buffer overflow at this port (note: this number does not include octets that are dropped because they arrived late had already been substituted by a filler octet). If the actual counter saturates, it remains on its maximum value. (R) (mandatory, if this voice port carries ISDN traffic) (4 bytes) |
| B2ChannelBuffer Overflows: | This attribute provides the total number of payload octets for the B2 channel that are dropped due to buffer overflow at this port (note: this number does not include octets that are dropped because they arrived late had already been substituted by a filler octet). If the actual counter saturates, it remains on its maximum value. (R) (mandatory, if this voice port carries ISDN traffic) (4 bytes) |
| DchannelBuffer Underflows: | This attribute provides the total number of filler octets injected into the D channel of the active media stream on this port due to playout buffer underflow. If the actual counter saturates, it remains on its maximum value. (R) (mandatory, if this voice port carries ISDN traffic) (4 bytes) |
| B1ChannelBuffer Overflows: | This attribute provides the total number of filler octets injected into the B1 channel of the active media stream on this port due to playout buffer underflow. If the actual counter saturates, it remains on its maximum value. (R) (mandatory, if this voice port carries ISDN traffic) (4 bytes) |
| B2ChannelBuffer Overflows: | This attribute provides the total number of filler octets injected into the B2 channel of the active media stream on this port due to playout buffer underflow. If the actual counter saturates, it remains on its maximum value. (R) (mandatory, if this voice port carries ISDN traffic) (4 bytes) |
| DchannelActiveSeconds: | This attribute provides the total length of time, in seconds, that the D channel associated with this port has been active. If the counter saturates, it remains on its maximum value. (R) (mandatory, if this voice port carries ISDN traffic) (4 bytes) |
| B1ChannelActiveSeconds: | This attribute provides the total length of time, in seconds, that the B1 channel associated with this port has been active. If the counter saturates, it remains on its maximum value. (R) (mandatory, if this voice port carries ISDN traffic) (4 bytes) |
| B2ChannelActiveSeconds: | This attribute provides the total length of time, in seconds, that the B2 channel associated with this port has been active. If the counter saturates, it remains on its maximum value. (R) (mandatory, if this voice port carries ISDN traffic) (4 bytes) |

Actions

| Create: | Create an instance of this managed entity. |
| --- | --- |
| Delete: | Delete an instance of this managed entity. |
| Get: | Get one or more attributes. |
| Set: | Set one or more attributes. |

Notifications

| Threshold Crossing Alert: | This notification is used to notify the management system when a threshold crossing alert (TCA) is detected or cleared. The TCA change notification “on” will be sent at the crossing of the threshold by the actual counter; the TCA change notification “off” will be sent at the end of the 15 min period since that is when the actual counters are reset to 0x00. The event list for this entity is given in Table 3/G.983.8. Table 3/G.983.8 replaces Table 15-e/G.983.2. Modifications from Table 15-3/G.983.2 are indicated by underlined text. |
| --- | --- |

Table 3/G.983.8  Alarm list for Voice PM History Data

| Number | Event | Description |
| --- | --- | --- |
|  | Threshold Crossing Alert |  |
| 0 | Reserved |  |
| 1 | Reserved |  |
| 2 | VoicePortBufferOverflows | Exceeds threshold |
| 3 | VoicePortBufferUnderflows | Exceeds threshold |
| 4 | B1ChannelBufferOverflows | Exceeds threshold |
| 5 | B2ChannelBufferOverflows | Exceeds threshold |
| 6 | DchannelBufferUnderflows | Exceeds threshold |
| 7 | B1ChannelBufferOverflows | Exceeds threshold |
| 8 | B2ChannelBufferOverflows | Exceeds threshold |
| 9-255 | Reserved |  |

### UPC Disagreement Monitoring History Data

To support VC cross-connections, replace “VP Network CTPB-PON” in clause 7.5.4/G.983.2 with “VP Network CTPB-PON or VC Network CTPB-PON”.

## IP Router function management

### IP Port Configuration Data

This managed entity is used to organize data that is associated with IP address and IP port provisioning. Instances of this managed entity are created and deleted by request of the OLT.

Relationship

Zero or one instances of this managed entity may exist for each instance of the Interworking VCC Termination Point managed entity and a Subscriber Line Card managed entity classified as a native LAN type (e.g. Ethernet).

Attributes

| Managed Entity id: | This attribute provides a unique number for each instance of this managed entity. (R, Set-by-create) (mandatory) (2 bytes) |
| --- | --- |
| PortNum: | This attribute identifies the port at which IP provisioning takes place. (R, Set-by-create) (mandatory) (1 byte) |
| TPType: | This attribute identifies the type of the termination point associated with this IP port. The value is set to 0x01 if this IP port is associated with the LAN side’s TP. The value is set to 0x02 if this IP port is associated with the ATM side’s TP. (R, Set-by-create) (mandatory) (1 byte) |
| TPPointer: | This attribute points to the instance of the TP associated with this port. When this IP port is associated with the LAN side’s TP, this attribute points to the instance of the PPTP Ethernet UNI. When this IP port is associated with the ATM side’s TP, this attribute points to the instance of the Interworking VCC Termination Point. (R, Set-by-create) (mandatory) (2 bytes) |
| PortAddress: | This attribute identifies the provisioned IP address. When an IP address is not assigned, this attribute consists of all 0x00. (R, Set-by-create) (mandatory) (4 bytes) |
| PortMask: | This attribute identifies the address mask associated with the provisioned IP address. When an IP address mask is not assigned, this attribute consists of all 0x00. (R, Set-by-create) (mandatory) (4 bytes) |
| Unnumbered: | This boolean attribute indicates whether or not this IP port has an IP address. The value TRUE means unnumbered. (For future use by IP router function.) (R, Set-by-create) (mandatory) (1 byte) |
| AdministrativeState: | This boolean attribute is used to activate (unlock, value 0x00) and deactivate (lock, value 0x01) the functions performed by instances of this managed entity. (R, W, Set-by-create) (1 byte) |
| PortState: | This boolean attribute provides status information on the port. Valid values include “up” (value 0x00) and “down” (value 0x01). (R, Set-by-create) (mandatory) (1 byte) |
| AllowRemoteAccess: | This boolean attribute indicates whether or not this IP port may be accessed remotely. The value TRUE means remote access is enabled. (R, Set-by-create) (mandatory) (1 byte) |
| Router Id Pointer: | This attribute identifies the IP router controlling the port, if it exists. This attribute points to the instance of the IP Router Service Profile. If no IP router controls the port, this attribute consists of a null pointer. (R, Set-by-create) (mandatory) (2 bytes) |
| ARP Pointer: | This attribute identifies the instance of ARP Service Profile associated with the IP port. (R, Set-by-create) (mandatory) (2 bytes) |
| EncapsulationMethod: | This attribute identifies the frame encapsulation method that is used. Value 0x00: identification by ATM VC; value 0x01: LLC encapsulation. Upon autonomous instantiation, the value 0x00 is used. (R. W) (mandatory) (1 byte) |

Actions

| Create: | Create an instance of this managed entity. |
| --- | --- |
| Delete: | Delete an instance of this managed entity. |
| Get: | Get one or more attributes. |
| Set: | Set one or more attributes. |

Notifications

| None. |  |
| --- | --- |

### IP Router Service Profile

This managed entity is used to organize data that affects all ports on an IP router on a routed LAN UNI interface on the ONT. Instances of this managed entity are created and deleted by request of the OLT.

Relationship

Zero or more instances of this managed entity shall be contained in an instance of a Subscriber Line Card managed entity classified as a native LAN type (e.g., Ethernet).

Attributes

| Managed Entity id: | This attribute provides a unique number for each instance of this managed entity. The first byte is the slot id (defined in 7.1.3/G.983.2). If the UNI is integrated, this value is 0x00. The second byte is the router group id. (R, Set-by-create) (mandatory) (2 bytes) |
| --- | --- |
| ForwardingInd: | This boolean attribute indicates whether or not forwarding of IP packets as a whole is enabled. The value TRUE means enabled. The initial value is enabled. (R, W, Set-by-create) (mandatory) (1 byte) |
| ProxyARPInd: | This boolean attribute indicates whether or not proxy ARP is enabled. The value TRUE means enabled. The initial value is enabled. (R, W, Set-by-create) (mandatory) (1 byte) |
| DirectedBroadcastInd: | This boolean attribute indicates whether or not relaying of directed broadcast packets is enabled. The value TRUE means enabled. The initial value is disabled. (R, W, Set-by-create) (mandatory) (1 byte) |
| UpstreamMulticast Filtering: | This attribute indicates whether upstream IP multicast packets are forwarded (value 0x00) or filtered (value 0x01). The initial value is 0x01. (R,W, Set-by-create) (mandatory) (1 byte) |
| DownstreamMulticast Filtering: | This attribute indicates whether downstream IP multicast packets are forwarded (value 0x00) or filtered (value 0x01). The initial value is 0x01. (R,W, Set-by-create) (mandatory) (1 byte) |

Actions

| Create: | Create an instance of this managed entity. |
| --- | --- |
| Delete: | Delete an instance of this managed entity. |
| Get: | Get one or more attributes. |
| Set: | Set one or more attributes. |

Notifications

| None. |  |
| --- | --- |

### IP Router Configuration Data

This managed entity is used to record data that is associated with IP router configurations. An instance of this managed entity shall be automatically created/deleted by the ONT upon creation/deletion of the associated instance of IP Router Service Profile.

Relationship

An instance of this managed entity is associated with one instance of IP Router Service Profile.

Attributes

| Managed Entity id: | This attribute provides a unique number for each instance of this managed entity. The assigned number is the same as the id of the IP Router Service Profile with which this IP Router Configuration Data is associated. (R) (mandatory) (2 bytes) |
| --- | --- |
| IpReasmTimeout: | This attribute indicates the maximum number of seconds that received fragments are held while they are awaiting reassembly at this router. (R) (mandatory) (4 bytes) |

Actions

| Get: | Get one or more attributes. |
| --- | --- |

Notifications

| None. |  |
| --- | --- |

### IP Router PM History Data1

This managed entity contains some IP-related past performance monitoring data collected at IP router function. Instances of this managed entity are created/deleted by the OLT after/before an instance of IP Router Service Profile is created/deleted.

Relationship

An instance of this managed entity is associated with one instance of IP Router Service Profile.

Attributes

| Managed Entity id: | This attribute provides a unique number for each instance of this managed entity. The assigned number is the same as the id of the IP Router Service Profile with which this IP Router PM History Data1 is associated. (R, Set-by-create) (mandatory) (2 bytes) |
| --- | --- |
| Interval End Time: | This attribute identifies the most recently finished 15-minute interval. It is a cyclic counter (modulo 0xFF (256)) that is incremented each time a new interval is finished and the attribute counters are updated. The value of this attribute is 0x00 during the first 15-minute interval that starts with the reception of the "synchronize time" action. The value is 0x01 during the first period after this, and so on. If this managed entity is created after the reception of the "synchronize time" action, the value of this attribute is set equal to the number of the last completed interval. The actual counters of this managed entity start counting directly. The attribute counters are updated at the end of the interval. (R) (mandatory) (1 byte) |
| Threshold DataB-PON id: | This attribute provides a pointer to an instance of the Threshold DataB-PON managed entity that contains the threshold values for the performance monitoring data collected by this managed entity. (R, W, Set-by-create) (mandatory) (2 bytes) |
| IpInReceivesCounter: | This attribute provides a count of packets received from interfaces, including those with errors. If the actual counter saturates, it remains on its maximum value. Default value is 0x00. (R) (mandatory) (4 bytes) |
| IpInHdrErrorsCounter: | This attribute provides a count of packets discarded due to errors in their IP headers, including bad checksums, version number mismatch, other format errors, time-to-live exceeded, errors discovered in processing their IP options, etc. If the actual counter saturates, it remains on its maximum value. Default value is 0x00. (R) (mandatory) (4 bytes) |
| IpInAddrErrorsCounter: | This attribute provides a count of packets discarded because the IP address in their IP header's destination field was not a valid address to be received at this router. This counter includes invalid addresses (e.g., 0.0.0.0) and addresses of unsupported Classes (e.g., Class E). If the actual counter saturates, it remains on its maximum value. Default value is 0x00. (R) (mandatory) (4 bytes) |
| IpForwPacketsCounter: | This attribute provides a count of packets for which this router was not their final IP destination, as a result of which an attempt was made to find a route to forward them to that final destination. If the actual counter saturates, it remains on its maximum value. Default value is 0x00. (R) (mandatory) (4 bytes) |
| IpInUnknownProtos Counter: | This attribute provides a count of packets which were locally-addressed and received successfully but discarded because of an unknown or unsupported protocol. If the actual counter saturates, it remains on its maximum value. Default value is 0x00. (R) (mandatory) (4 bytes) |
| IpInDiscardsCounter: | This attribute provides a count of input packets for which no problems were encountered to prevent their continued processing, but which were discarded (e.g., for lack of buffer space). Note that this counter does not include any packets discarded while awaiting re-assembly. If the actual counter saturates, it remains on its maximum value. Default value is 0x00. (R) (mandatory) (4 bytes) |
| IpInDeliversCounter: | This attribute provides a count of input packets successfully delivered to IP user-protocols (including ICMP). If the actual counter saturates, it remains on its maximum value. Default value is 0x00. (R) (mandatory) (4 bytes) |
| IpOutRequestsCounter: | This attribute provides a count of packets that local IP user-protocols (including ICMP) supplied to IP in requests for transmission. Note that this counter does not include any packets counted in ipForwPacketsCounter. If the actual counter saturates, it remains on its maximum value. Default value is 0x00. (R) (mandatory) (4 bytes) |
| IpOutDiscardsCounter: | This attribute provides a count of output packets for which no problem was encountered to prevent their transmission to their destination, but which were discarded (e.g., for lack of buffer space). Note that this counter would include packets counted in ipForwPacketsCounter if any such packets met this (discretionary) discard criterion. If the actual counter saturates, it remains on its maximum value. Default value is 0x00. (R) (mandatory) (4 bytes) |
| IpOutNoRoutesCounter: | This attribute provides a count of packets discarded because no route could be found to transmit them to their destination. Note that this counter includes any packets counted in ipForwPacketsCounter which meet this `no-route' criterion. Note that this includes any packets that a host cannot route because all of its default gateways are down. If the actual counter saturates, it remains on its maximum value. Default value is 0x00. (R) (mandatory) (4 bytes) |

Actions

| Create: | Create an instance of this managed entity. |
| --- | --- |
| Delete: | Delete an instance of this managed entity. |
| Get: | Get one or more attributes. |
| Set: | Set one or more attributes. |

Notifications

| Threshold Crossing Alert: | This notification is used to notify the management system when a Threshold Crossing Alert (TCA) has been detected or cleared. The TCA change notification "on" will be sent at the crossing of the threshold by the actual counter; the TCA change notification "off" will be sent at the end of the 15 min period since that is when the actual counters are reset to 0x00. The event list for this entity is given in Table 4/G.983.8. |
| --- | --- |

Table 4/G.983.8  Alarm list for IP Router PM History Data1

| Number | Event | Description |
| --- | --- | --- |
|  | Threshold Crossing Alert |  |
| 0 | IpInReceives | Exceeds threshold |
| 1 | IpInHdr | Exceeds threshold |
| 2 | IpInAddr | Exceeds threshold |
| 3 | IpForwPackets | Exceeds threshold |
| 4 | IpInUnknownProtos | Exceeds threshold |
| 5 | IpInDiscards | Exceeds threshold |
| 6 | IpInDelivers | Exceeds threshold |
| 7 | IpOutRequests | Exceeds threshold |
| 8 | IpOutDiscards | Exceeds threshold |
| 9 | IpOutNoRoutes | Exceeds threshold |
| 10-255 | Reserved |  |

### IP Router PM History Data2

This managed entity contains other IP-related past performance monitoring data collected at IP router function. Instances of this managed entity are created/deleted by the OLT after/before an instance of IP Router Service Profile is created/deleted.

Relationship

An instance of this managed entity is associated with one instance of IP Router Service Profile.

Attributes

| Managed Entity id: | This attribute provides a unique number for each instance of this managed entity. The assigned number is the same as the id of the IP Router Service Profile with which this IP Router PM History Data2 is associated. (R, Set-by-create) (mandatory) (2 bytes) |
| --- | --- |
| Interval End Time: | This attribute identifies the most recently finished 15-minute interval. It is a cyclic counter (modulo 0xFF (256)) that is incremented each time a new interval is finished and the attribute counters are updated. The value of this attribute is 0x00 during the first 15-minute interval that starts with the reception of the "synchronize time" action. The value is 0x01 during the first period after this, and so on. If this managed entity is created after the reception of the "synchronize time" action, the value of this attribute is set equal to the number of the last completed interval. The actual counters of this managed entity start counting directly. The attribute counters are updated at the end of the interval. (R) (mandatory) (1 byte) |
| Threshold DataB-PON id: | This attribute provides a pointer to an instance of the Threshold DataB-PON managed entity that contains the threshold values for the performance monitoring data collected by this managed entity. (R, W, Set-by-create) (mandatory) (2 bytes) |
| IpReasmReqdsCounter: | This attribute provides a count of received packets that needed to be reassembled at this entity. Default value is 0x00. (R) (mandatory) (4 bytes) |
| IpReasmOKsCounter: | This attribute provides a count of packets successfully re-assembled. Default value is 0x00. (R) (mandatory) (4 bytes) |
| IpReasmFailsCounter: | This attribute provides a count of failures detected by the IP re-assembly algorithm (for whatever reason: timed out, errors, etc). Note that this is not necessarily a count of discarded IP fragments since some algorithms (notably the algorithm in RFC 815) can lose track of the number of fragments by combining them as they are received. Default value is 0x00. (R) (mandatory) (4 bytes) |
| IpFragOKsCounter: | This attribute provides a count of packets that have been successfully fragmented at this entity. Default value is 0x00. (R) (mandatory) (4 bytes) |
| IpFragFailsCounter: | This attribute provides a count of packets that have been discarded because they needed to be fragmented at this router but could not be, e.g., because their Don't Fragment flag was set. Default value is 0x00. (R) (mandatory) (4 bytes) |
| IpFragCreatesCounter: | This attribute provides a count of IP fragments that have been generated as a result of fragmentation at this entity. Default value is 0x00. (R) (mandatory) (4 bytes) |

Actions

| Create: | Create an instance of this managed entity. |
| --- | --- |
| Delete: | Delete an instance of this managed entity. |
| Get: | Get one or more attributes. |
| Set: | Set one or more attributes. |

Notifications

| Threshold Crossing Alert: | This notification is used to notify the management system when a Threshold Crossing Alert (TCA) has been detected or cleared. The TCA change notification "on" will be sent at the crossing of the threshold by the actual counter; the TCA change notification "off" will be sent at the end of the 15 min period since that is when the actual counters are reset to 0x00. The event list for this entity is given in Table 5/G.983.8. |
| --- | --- |

Table 5/G.983.8  Alarm list for IP Router PM History Data2

| Number | Event | Description |
| --- | --- | --- |
|  | Threshold Crossing Alert |  |
| 0 | IpReasmReqds | Exceeds threshold |
| 1 | IpReasmOKs | Exceeds threshold |
| 2 | IpReasmFails | Exceeds threshold |
| 3 | IpFragOKs | Exceeds threshold |
| 4 | IpFragFails | Exceeds threshold |
| 5 | IpFragCreates | Exceeds threshold |
| 6-255 | Reserved |  |

### ICMP PM History Data1

This managed entity contains some ICMP-related past performance monitoring data collected at IP router function. Instances of this managed entity are created/deleted by the OLT after/before an instance of IP Router Service Profile is created/deleted.

Relationship

An instance of this managed entity is associated with one instance of IP Router Service Profile.

Attributes

| Managed Entity id: | This attribute provides a unique number for each instance of this managed entity. The assigned number is the same as the id of the IP Router Service Profile with which this ICMP PM History Data1 is associated. (R, Set-by-create) (mandatory) (2 bytes) |
| --- | --- |
| Interval End Time: | This attribute identifies the most recently finished 15-minute interval. It is a cyclic counter (modulo 0xFF (256)) that is incremented each time a new interval is finished and the attribute counters are updated. The value of this attribute is 0x00 during the first 15-minute interval that starts with the reception of the "synchronize time" action. The value is 0x01 during the first period after this, and so on. If this managed entity is created after the reception of the "synchronize time" action, the value of this attribute is set equal to the number of the last completed interval. The actual counters of this managed entity start counting directly. The attribute counters are updated at the end of the interval. (R) (mandatory) (1 byte) |
| Threshold DataB-PON id: | This attribute provides a pointer to an instance of the Threshold DataB-PON managed entity that contains the threshold values for the performance monitoring data collected by this managed entity. (R, W, Set-by-create) (mandatory) (2 bytes) |
| IcmpInMsgsCounter: | This attribute provides a count of received ICMP messages. Note that this counter includes all those counted by IcmpInErrorsCounter. Default value is 0x00. (R) (mandatory) (4 bytes) |
| IcmpInErrorsCounter: | This attribute provides a count of ICMP messages that were received but determined as having ICMP-specific errors (bad ICMP checksums, bad length, etc.). Default value is 0x00. (R) (mandatory) (4 bytes) |
| IcmpInDestUnreachs Counter: | This attribute provides a count of received ICMP Destination Unreachable messages. Default value is 0x00. (R) (mandatory) (4 bytes) |
| IcmpInTimeExcds Counter: | This attribute provides a count of received ICMP Time Exceeded messages. Default value is 0x00. (R) (mandatory) (4 bytes) |
| IcmpInParmProbs Counter: | This attribute provides a count of received ICMP Parameter Problem messages. Default value is 0x00. (R) (mandatory) (4 bytes) |
| IcmpInSrcQuenchs Counter: | This attribute provides a count of received ICMP Source Quench messages. Default value is 0x00. (R) (mandatory) (4 bytes) |
| IcmpInRedirects Counter: | This attribute provides a count of received ICMP Redirect messages. Default value is 0x00. (R) (mandatory) (4 bytes) |
| IcmpInEchosCounter: | This attribute provides a count of received ICMP Echo (request) messages. Default value is 0x00. (R) (mandatory) (4 bytes) |
| IcmpInEchoReps Counter: | This attribute provides a count of received ICMP Echo Reply messages. Default value is 0x00. (R) (mandatory) (4 bytes) |
| IcmpInTimestamps Counter: | This attribute provides a count of received ICMP Timestamp (request) messages. Default value is 0x00. (R) (mandatory) (4 bytes) |
| IcmpInTimestampRepsCounter: | This attribute provides a count of received ICMP Timestamp Reply messages. Default value is 0x00. (R) (mandatory) (4 bytes) |
| IcmpInAddrMasks Counter: | This attribute provides a count of received ICMP Address Mask (request) messages. Default value is 0x00. (R) (mandatory) (4 bytes) |
| IcmpInAddrMaskRepsCounter: | This attribute provides a count of received ICMP Address Mask Reply messages. Default value is 0x00. (R) (mandatory) (4 bytes) |

Actions

| Create: | Create an instance of this managed entity. |
| --- | --- |
| Delete: | Delete an instance of this managed entity. |
| Get: | Get one or more attributes. |
| Set: | Set one or more attributes. |

Notifications

| Threshold Crossing Alert: | This notification is used to notify the management system when a Threshold Crossing Alert (TCA) has been detected or cleared. The TCA change notification "on" will be sent at the crossing of the threshold by the actual counter; the TCA change notification "off" will be sent at the end of the 15 min period since that is when the actual counters are reset to 0x00. The event list for this entity is given in Table 6/G.983.8. |
| --- | --- |

Table 6/G.983.8  Alarm list for ICMP PM History Data1

| Number | Event | Description |
| --- | --- | --- |
|  | Threshold Crossing Alert |  |
| 0 | IcmpInMsgs | Exceeds threshold |
| 1 | IcmpInErrors | Exceeds threshold |
| 2 | IcmpInDestUnreachs | Exceeds threshold |
| 3 | IcmpInTimeExcds | Exceeds threshold |
| 4 | IcmpInParmProbs | Exceeds threshold |
| 5 | IcmpInSrcQuenchs | Exceeds threshold |
| 6 | IcmpInRedirects | Exceeds threshold |
| 7 | IcmpInEchos | Exceeds threshold |
| 8 | IcmpInTimestamps | Exceeds threshold |
| 9 | IcmpInTimestampReps | Exceeds threshold |
| 10 | IcmpInAddrMasks | Exceeds threshold |
| 11 | IcmpInAddrMaskReps | Exceeds threshold |
| 12-255 | Reserved |  |

### ICMP PM History Data2

This managed entity contains other ICMP-related past performance monitoring data collected at IP router function. Instances of this managed entity are created/deleted by the OLT after/before an instance of IP Router Service Profile is created/deleted.

Relationship

An instance of this managed entity is associated with one instance of IP Router Service Profile.

Attributes

| Managed Entity id: | This attribute provides a unique number for each instance of this managed entity. The assigned number is the same as the id of the IP Router Service Profile with which this ICMP PM History Data2 is associated. (R, Set-by-create) (mandatory) (2 bytes) |
| --- | --- |
| Interval End Time: | This attribute identifies the most recently finished 15-minute interval. It is a cyclic counter (modulo 0xFF (256)) that is incremented each time a new interval is finished and the attribute counters are updated. The value of this attribute is 0x00 during the first 15-minute interval that starts with the reception of the "synchronize time" action. The value is 0x01 during the first period after this, and so on. If this managed entity is created after the reception of the "synchronize time" action, the value of this attribute is set equal to the number of the last completed interval. The actual counters of this managed entity start counting directly. The attribute counters are updated at the end of the interval. (R) (mandatory) (1 byte) |
| Threshold DataB-PON id: | This attribute provides a pointer to an instance of the Threshold DataB-PON managed entity that contains the threshold values for the performance monitoring data collected by this managed entity. (R, W, Set-by-create) (mandatory) (2 bytes) |
| IcmpOutMsgsCounter: | This attribute provides a count of ICMP messages that this router attempted to send. Note that this counter includes all those counted by IcmpOutErrorsCounter. Default value is 0x00. (R) (mandatory) (4 bytes) |
| IcmpOutErrorsCounter: | This attribute provides a count of ICMP messages that this router did not send due to problems discovered within ICMP such as a lack of buffers. This value should not include errors discovered outside the ICMP layer such as the inability of IP to route the resultant packet. In some implementations, there may be no types of error that contribute to this counter's value. Default value is 0x00. (R) (mandatory) (4 bytes) |
| IcmpOutDestUnreachs Counter: | This attribute provides a count of sent ICMP Destination Unreachable messages. Default value is 0x00. (R) (mandatory) (4 bytes) |
| IcmpOutTimeExcds Counter: | This attribute provides a count of sent ICMP Time Exceeded messages. Default value is 0x00. (R) (mandatory) (4 bytes) |
| IcmpOutParmProbs Counter: | This attribute provides a count of sent ICMP Parameter Problem messages. Default value is 0x00. (R) (mandatory) (4 bytes) |
| IcmpOutSrcQuenchs Counter: | This attribute provides a count of sent ICMP Source Quench messages. Default value is 0x00. (R) (mandatory) (4 bytes) |
| IcmpOutRedirects Counter: | This attribute provides a count of sent ICMP Redirect messages. Default value is 0x00. (R) (mandatory) (4 bytes) |
| IcmpOutEchosCounter: | This attribute provides a count of sent ICMP Echo (request) messages. Default value is 0x00. (R) (mandatory) (4 bytes) |
| IcmpOutEchoReps Counter | This attribute provides a count of sent ICMP Echo Reply messages. Default value is 0x00. (R) (mandatory) (4 bytes) |
| IcmpOutTimestamps Counter: | This attribute provides a count of sent ICMP Timestamp (request) messages. Default value is 0x00. (R) (mandatory) (4 bytes) |
| IcmpOutTimestampRepsCounter: | This attribute provides a count of sent ICMP Timestamp Reply messages. Default value is 0x00. (R) (mandatory) (4 bytes) |
| IcmpOutAddrMasks Counter: | This attribute provides a count of sent ICMP Address Mask (request) messages. Default value is 0x00. (R) (mandatory) (4 bytes) |
| IcmpOutAddrMaskRepsCounter: | This attribute provides a count of sent ICMP Address Mask Reply messages. Default value is 0x00. (R) (mandatory) (4 bytes) |

Actions

| Create: | Create an instance of this managed entity. |
| --- | --- |
| Delete: | Delete an instance of this managed entity. |
| Get: | Get one or more attributes. |
| Set: | Set one or more attributes. |

Notifications

| Threshold Crossing Alert: | This notification is used to notify the management system when a Threshold Crossing Alert (TCA) has been detected or cleared. The TCA change notification "on" will be sent at the crossing of the threshold by the actual counter; the TCA change notification "off" will be sent at the end of the 15 min period since that is when the actual counters are reset to 0x00. The event list for this entity is given in Table 7/G.983.8. |
| --- | --- |

Table 7/G.983.8  Alarm list for ICMP PM History Data2

| Number | Event | Description | Threshold Data Counter #* |
| --- | --- | --- | --- |
|  | Threshold Crossing Alert |  |  |
| 0 | IcmpOutMsgs | Exceeds threshold | 1 |
| 1 | IcmpOutErrors | Exceeds threshold | 2 |
| 2 | IcmpOutDestUnreachs | Exceeds threshold | 3 |
| 3 | IcmpOutTimeExcds | Exceeds threshold | 4 |
| 4 | IcmpOutParmProbs | Exceeds threshold | 5 |
| 5 | IcmpOutSrcQuenchs | Exceeds threshold | 6 |
| 6 | IcmpOutRedirects | Exceeds threshold | 7 |
| 7 | IcmpOutEchos | Exceeds threshold | 8 |
| 8 | IcmpOutTimestamps | Exceeds threshold | 9 |
| 9 | IcmpOutTimestampReps | Exceeds threshold | 10 |
| 10 | IcmpOutAddrMasks | Exceeds threshold | 11 |
| 11 | IcmpOutAddrMaskReps | Exceeds threshold | 12 |
| 12-255 | Reserved |  |  |

*This numbering is used with the associated Threshold Data B-PON managed entity.  Threshold Data counter 1 indicates the 1st thresholded counter, etc.

### IP Route Table

This managed entity is used to record data that is associated with IP routes. Some of the data is volatile. Instances of this managed entity shall be automatically created/deleted by the ONT upon creation/deletion of the associated instances of IP Router Service Profile.

Relationship

An instance of this managed entity is associated with one instance of IP Router Service Profile.

Attributes

| Managed Entity id: | This attribute provides a unique number for each instance of this managed entity. The assigned number is the same as the id of the IP Router Service Profile with which this IP Route Table is associated. (R) (mandatory) (2 bytes) |
| --- | --- |
| IpRouteNumber: | This attribute indicates the number of current routes in the route table. Default value is 0x00. (R) (mandatory) (2 bytes) |
| IpRouteTableMaxSize: | This attribute indicates the maximum number of routes that can be stored in the route table. (R) (mandatory) (2 bytes) |
| IpRouteTable: | This attribute lists current routes in the route table. A route consists of route-id, destination-address, destination-address-mask, tos, next-hop, output-port, route-type, route-protocol, route-age, and metric, which are further described below. (R) (mandatory) (N * 30 bytes. N is the number of routes.) route-id: a unique identifier of a route within the route table (2 bytes); destination-address: the destination IP address of this route (4 bytes); destination-address-mask: the address mask associated with the destination address (4 bytes); tos: TOS value defined in RFC 2096 [3] (ipCidrRouteTos) for use with policy routing; otherwise, 0; (1 byte) next-hop: the address of the next router on remote routes (4 bytes); output-port: the port number of the IP port through which the next hop of this route should be reached (1 byte); route-type: the type of route as defined in RFC 2096 [3] (ipCidrRouteType) (1 byte); route-protocol: the routing mechanism via which this route was learned, as defined in RFC 2096 [3] (ipCidrRouteProto). For example, static route is 3 (1 byte); route-age: the number of seconds since this route was last updated or otherwise determined to be correct. Static routes can return the maximum value (4 bytes); metric: the primary and secondary routing metrics for this route. The semantics of this metric are determined by the routing-protocol specified in the route's route-protocol value. If part of this metric is not used, its value should be set to –1 (2* 4 bytes). |

Actions

| Get: | Get one or more attributes. Latch a snapshot (i.e. copy) of the current ipRouteTable and respond with the size of data (4 bytes) that should be obtained using the Get next command. |
| --- | --- |
| Get next: | Get the latched attribute values of the managed entity within the current snapshot. |

Notifications

| None. |  |
| --- | --- |

### IP Static Routes

This managed entity is used to set or delete IP static routes. Instances of this managed entity shall be automatically created/deleted by the ONT upon creation/deletion of the associated instances of IP Router Service Profile.

Relationship

An instance of this managed entity is associated with one instance of IP Router Service Profile.

Attributes

| Managed Entity id: | This attribute provides a unique number for each instance of this managed entity. The assigned number is the same as the id of the IP Router Service Profile with which this IP Static Routes is associated. (R) (mandatory) (2 bytes) |
| --- | --- |
| IpStaticRouteTableMaxSize: | This attribute indicates the maximum number of routes that can be set as static routes. (R) (mandatory) (2 bytes) |
| IpStaticRouteTable: | This attribute is used to set or delete static routes in the route table. A route consists of route-id, action, destination-address, destination-address-mask, next-hop, output-port, and metric, which are further described below. (The same scenario is used for the MAC Filter Table attribute of the MAC Bridge Port Filter Table Data managed entity. See I.1.3 and I.1.4.) When this attribute is used to set a static route through the parameter action:add(1), the route table is searched for the route-id parameter associated with the static route that is to be added. If the same route-id is found, the route is overwritten. (R, W) (mandatory) (N * 21 bytes. N is the number of routes.) route-id: a unique identifier of a route within the static route table (1 byte); action: remove (0) or add (1) this route. When a static route is being removed, only static-route-id field is used to identify the route (1 byte); destination-address: the destination IP address of this route. This field may be set at the default route address (0.0.0.0) (4 bytes); destination-address-mask: the address mask associated with the destination address (4 bytes); next-hop: the address of the next router on remote routes. This field is not used when the next-hop is reached through an unnumbered link (4 bytes, 0xFFFFFFFF when not used); output-port: the port number of the IP port through which the next hop of this route should be reached. This field is used when the next-hop is reached through an unnumbered link and when a static route is set to support a fully meshed network over ATM (1 byte, 0xFF when not used); iVCCTPointer: the pointer to the instance of Interworking VCC Termination Point which identifies one of the ATM links of the fully meshed network which is being set by this attribute (optional, 2 bytes); metric: the routing metric for this route (4 bytes). |

Actions

| Get: | Get one or more attributes. Latch a snapshot (i.e. copy) of the current IpStaticRouteTable and respond with the size of data (4 bytes) that should be obtained using the Get next command. |
| --- | --- |
| Set: | Set one or more attributes. |
| Get next: | Get the latched attribute values of the managed entity within the current snapshot. |

Notifications

| None. |  |
| --- | --- |

### ARP Service Profile

This managed entity is used to organize data that is associated with the ARP function used by an IP router function that contains an IP port of native LAN type. Instances of this managed entity are created/deleted by the OLT after/before an instance of IPPortConfigurationData is created/deleted.

Relationship

An instance of this managed entity is associated with one instance of IP Port Configuration Data of native LAN type.

Attributes

| Managed Entity id: | This attribute provides a unique number for each instance of this managed entity. The first byte is the slot id (defined in 7.1.3/G.983.2). If the UNI is integrated, this value is 0x00. The second byte is the ARP group id. (R, Set-by-create) (mandatory) (2 bytes) |
| --- | --- |
| ARP Timer: | This attribute indicates the maximum number of seconds that IP packets are held while they are awaiting address resolution by ARP at this router. (R) (mandatory) (4 bytes) |

| ARP Cache Clear: | This boolean attribute is used by the management system to initialize the ARP cache associated with an instance of this entity. This attribute is used only to trigger the “Cache Clear” action. The value TRUE means clear, and the value FALSE has no significance. As the value of this attribute has no physical meaning, the action “Get” will always return the value FALSE for this attribute. (R, W, Set-by-create) (mandatory) (1 byte) |
| --- | --- |

Actions

| Create: | Create an instance of this managed entity. |
| --- | --- |
| Delete: | Delete an instance of this managed entity. |
| Get: | Get one or more attributes. |
| Set: | Set one or more attributes. |

Notifications

| None. |  |
| --- | --- |

### ARP Configuration Data

This managed entity is used to organize data that is associated with the ARP function related to an IP router that contains an IP port of native LAN type. Some of the data is volatile. Instances of this managed entity shall be automatically created/deleted by the ONT upon creation/deletion of the associated instance of ARP Service Profile.

Relationship

An instance of this managed entity is associated with one instance of ARP Service Profile.

Attributes

| Managed Entity id: | This attribute provides a unique number for each instance of this managed entity. The assigned number is the same as the id of the ARP Service Profile with which this ARP Configuration Data is associated. (R) (mandatory) (2 bytes) |
| --- | --- |
| ARPTableMaxSize: | This attribute indicates the maximum number of ARP entries that can be set in the ARP table. (R) (mandatory) (2 bytes) |
| ARP Table: | This attribute lists current entries in the ARP cache. An ARP cache entry consists of port, IP-address, MAC-address, and type, which are further described below. (R) (mandatory) (N * 12 bytes. N is the number of entries.) port: the port number of the IP port associated with the entry (1 byte); IP-address: an IP address (4 bytes); MAC-address: an equivalent MAC-address resolved by the ARP procedure (6 bytes); type: the entry type such as dynamic (3) or static (4) (1 byte). |

Actions

| Get: | Get one or more attributes. Latch a snapshot (i.e. copy) of the current ARP Table and respond with the size of data (4 bytes) that should be obtained using the Get next command. |
| --- | --- |
| Get next: | Get the latched attribute values of the managed entity within the current snapshot. |

Notifications

| None. |  |
| --- | --- |

## ISDN management

### Physical Path Termination Point ISDN UNI

This managed entity represents the point at the ISDN UNI in the ONT where physical paths terminate and physical path level functions (e.g. analog telephony, facsimile function) are performed.  An instance of this managed entity shall be automatically created/deleted by the ONT upon the creation/deletion of a Subscriber Line Card with ISDN type.

Relationships

One or more instances of this managed entity shall be contained in an instance of the ONTB-PON or a Subscriber Line Card managed entity classified as ISDN type.

Attributes

| Managed Entity id: | This attribute provides a unique number for each instance of this managed entity. This 2-byte number is directly associated with the physical position of the UNI. The first byte is the slot id (defined in 7.1.3). If the UNI is integrated, this value is 0x00. The second byte is the port id with value range from 0x01 to 0xFF (1 to 255), 0x01 is used for the leftmost/lowest port on a Subscriber Line Card, 0x02 is used for the next right/upper port, and so forth. (R) (mandatory) (2 bytes) |
| --- | --- |
| Administrative State: | This attribute is used to activate (unlock: value 0x00) and deactivate (lock: value 0x01) the functions performed by instances of this managed entity. Selection of a default value for this attribute is outside the scope of this document as it is normally handled through supplier-operator negotiations. (R, W) (mandatory) (1 byte) |
| Interworking VCC Pointer: | This attribute provides a pointer to the instance of the Interworking VCC Termination Point managed entity to which this instance is connected. (R, W) (optional) (2 bytes) |
| DchannelID: | This attribute provides the channel identifier of the AAL2 connection transporting the D channel associated with this ISDN BRI port. (R, set-by-create) (mandatory) (1 byte) |
| B1ChannelID: | This attribute provides the channel identifier of the AAL2 connection transporting the B1 channel associated with this ISDN BRI port. (R, set-by-create) (mandatory) (1 byte) |
| B2ChannelID: | This attribute provides the channel identifier of the AAL2 connection transporting the B2 channel associated with this ISDN BRI port. (R, set-by-create) (mandatory) (1 byte) |

| ARC: | This attribute is used to control alarm reporting from this managed entity. Valid values are “off” (alarm reporting allowed immediately, value 0x00) and “on” (alarm reporting inhibited, value 0x01). Upon initial installation and provisioning of the ONT, this attribute may be set to “on” or “off” for the time interval specified by “ARCInterval.” Similarly, this attribute may be set to “off”. If the attribute is set to “on”, then alarm reporting is inhibited until this managed entity detects a valid signal for the time interval specified by “ARCInterval.” (R, W) (optional) (1 byte) |
| --- | --- |
| ARCInterval: | This attribute provides a provisionable length of time. Units are given in minutes (R, W) (optional) (1 byte) |
| ISDN Loopback Configuration: | This attribute represents the loopback configuration of this physical interface. Value 0x00: no loopback; value 0x01: simultaneous loopback of all channels; value 0x01: loopback of D channel only; value 0x02: loopback of B1 channel only; value 0x03: loopback of B2 channel only. Upon autonomous instantiation, the value 0x00 is used. (R, W) (mandatory) (1 byte) |

Actions

| Get: | Get one or more attributes. |
| --- | --- |
| Set: | Set one or more attributes. |
| Test: | Request that the ONT perform one or more MLT tests. See “Test” and “Test result” message layouts in sections II.2.27/G.983.2 and II.28.45/G.983.2. |

Notifications

| Alarm: | This notification is used to notify the management system when a failure has been detected or cleared. Both ONT and OLT should know the alarm list used by this entity. The alarm list for this entity is given in Table 8/G.983.8. |
| --- | --- |

Table 8/G.983.8  Alarm list for Physical Path Termination Point ISDN UNI

| Number | Alarm | Description |
| --- | --- | --- |
| 0 | AIS | Alarm indication signal |
| 1 | RDI | Remote defect indication |
| 2-255 | Reserved |  |

## VLAN tagging management

### VLAN Tagging Operation Configuration Data

This managed entity is used to organize data associated with VLAN tagging. Instances of this managed entity are created/deleted at the request of the OLT.

Relationship

Zero or one instance of this managed entity may exist for each instance of Physical Path Termination Point Ethernet UNI.

Attributes

| Managed Entity id: | This attribute provides a unique number for each instance of this managed entity. The assigned number is the same as the id of the Physical Path Termination Point Ethernet UNI with which this VLAN Tagging Operation Configuration Data instance is associated. (R, Set-by-create) (mandatory) (2 bytes) |
| --- | --- |
| Upstream VLAN Tagging Operation Mode: | This attribute selects whether or not upstream VLAN tagging is sent. Valid values are 0x00 (upstream frame is sent “as is,” regardless of whether or not the received frame is tagged) and 0x01 (The upstream frame is sent as tagged whether or not the received frame is tagged. TCI, consisting of VID, CFI and user priority, is attached or overwritten by using the Upstream VLAN Tag TCI Value.). (R, W, Set-by-create) (mandatory) (1 byte) |
| Upstream VLAN Tag TCI Value | This attribute indicates the TCI value for upstream VLAN tagging. It is used when the Upstream VLAN Tagging Operation Mode is 0x01. Any 2-byte value is acceptable. (R, W, Set-by-create) (mandatory) (2 bytes) |
| Downstream VLAN Tagging Operation Mode: | This attribute selects whether or not downstream VLAN tagging is sent. Valid values are 0x00 (downstream frame is sent “as is,” regardless of whether or not the received frame is tagged) and 0x01 (The downstream frame is sent as untagged whether or not the received frame is tagged). (R, W, Set-by-create) (mandatory) (1 byte) |

Actions

| Create: | Create an instance of this managed entity. |
| --- | --- |
| Delete: | Delete an instance of this managed entity. |
| Get: | Get one or more attributes. |
| Set: | Set one or more attributes. |

Notifications

| None. |  |
| --- | --- |

### VLAN Tagging Filter Data

This managed entity is used to organize data associated with VLAN tagging.  Instances of this managed entity are created/deleted at the request of the OLT.

Relationships

Zero or one instance of this managed entity may exist for each instance of MAC Bridge Port Configuration Data which is associated with Interworking VCC Termination Point (PON side) or Physical Path Termination Point Ethernet UNI.

Attributes

| Managed Entity id: | This attribute provides a unique number for each instance of this managed entity. The assigned number is the same as the id of the MACBridgePortConfigurationData with which this VLAN Tagging Filter Data instance is associated. (R, Set-by-create) (mandatory) (2 bytes) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| VLAN Filter Table: | This attribute lists TCI values which are provisioned at a bridging port. Since a TCI value (consisting of user priority, CFI and VID) is represented by 16 bits, 2 bytes are reserved for this attribute per VLAN. (R, W, Set-by-create) (mandatory) (2 bytes* number of supported VLANs) |  |  |  |  |
| Forward Operation: | When a frame is received, the frame is processed according to the following Forward Operations. The operations are invoked based on the value of VID, user priority, or the entire TCI or whether or not the TCI field exists. This attribute indicates the received frame is treated as indicated below. (R,W, set-by-create) (mandatory) (1 byte) |  |  |  |  |
|  |  |  | Type of received frame |  |  |
|  |  |  | Tagged | Untagged |  |
|  |  | 0x00 | Depends on MAC Bridge Port Bridge Table Data | Depends on MAC Bridge Port Bridge Table Data |  |
|  |  | 0x01 | Discard | Depends on MAC Bridge Port Bridge Table Data |  |
|  |  | 0x02 | Depends on MAC Bridge Port Bridge Table Data | Discard |  |
|  |  | 0x03 | Depends on MAC Bridge Port Bridge Table Data when the received VID is included in this table. Discarded when the received VID is not included in this table. | Depends on MAC Bridge Port Bridge Table Data |  |
|  |  | 0x04 |  | Discard |  |
|  |  | 0x05 | Discarded when the received VID is included in this table. Depends on MAC Bridge Port Bridge Table Data when the received VID is not included in this table. | Depends on MAC Bridge Port Bridge Table Data |  |
|  |  | 0x06 |  | Discard |  |
|  |  | 0x07 | Depends on MAC Bridge Port Bridge Table Data when the received user priority is included in this table. Discarded when the received user priority is not included in this table. | Depends on MAC Bridge Port Bridge Table Data |  |
|  |  | 0x08 |  | Discard |  |
|  |  | 0x09 | Discarded when the received user priority is included in this table. Depends on MAC Bridge Port Bridge Table Data when the received user priority is not included in this table. | Depends on MAC Bridge Port Bridge Table Data |  |
|  |  | 0x0A |  | Discard |  |
|  |  | 0x0B | Depends on MAC Bridge Port Bridge Table Data when the received TCI is included in this table. Discarded when the entire received TCI is not included in this table. | Depending on MAC Bridge Port Bridge Table Data |  |
|  |  | 0x0C |  | Discard |  |
|  |  | 0x0D | Discarded when the entire received TCI is included in this table. Depends on MAC Bridge Port Bridge Table Data when the entire received TCI is not included in this table. | Depends on MAC Bridge Port Bridge Table Data |  |
|  |  | 0x0E |  | Discard |  |

Actions

| Create: | Create an instance of this managed entity. |
| --- | --- |
| Delete: | Delete an instance of this managed entity. |
| Get: | Get one or more attributes. |
| Set: | Set one or more attributes. |

Notifications

| None. |  |
| --- | --- |

Supplementary explanation

(1) Ethernet frame and fields format for VLAN services

The detailed format of Ethernet frame for VLAN services is described in IEEE 802.1Q [4] and is depicted in Figure 5/G.983.8.

Figure 5/G.983.8 – Detailed format for Ethernet frame with VLAN tag

(2) Operations

Operations specified in the attribute “Forward Operation” are explained. Forward Operations are specified by a combination of the basic actions mentioned.

- Basic MAC bridge operation: As shown in Figure 6/G.983.8, if DA (Destination MAC Address) in the received frame is listed in one or more ports with MAC Bridge Port Bridge Table Data, this frame is forwarded to the indicated ports. Otherwise, it is broadcasted to all of the ports excepting for its receiving port.

Figure 6/G.983.8 – Basic MAC Bridge operation

- Unconditional forwarding for Tagged frame: If a tagged frame is received, it is forwarded without investigation of TCI.

- Unconditional discarding for Tagged frame: If a tagged frame is received, it is discarded without investigation of TCI.

- Unconditional forwarding for Untagged frame: If an untagged frame is received, it is forwarded without investigation of TCI.

- Unconditional discarding for Untagged frame: If an untagged frame is received, it is discarded without investigation of TCI.

- Positive filtering by TCI: If a part or all of the fields in the TCI of the received frame are included in VLAN Tag Filter Data, it is forwarded according to Action (a) to the indicted ports as shown in Figure 7/G.983.8. Otherwise, its TCI is ignored and it is controlled by Action (a).

Figure 7/G.983.8 – Positive filtering by TCI operation

- Negative filtering by TCI: If a part or all of the fields in theTCI of the received frame are included in VLAN Tag Filter Data, it is discarded. Otherwise, it is forwarded according to Action (a) as shown in Figure 8/G.983.8.

Figure 8/G.983.8 – Negative filtering by TCI operation

The Forwarding Operation is specified by combination of these actions.

|  | The type of received frame |  |
| --- | --- | --- |
|  | Tagged | Untagged |
| 0x00 | Action (a) | Action (a) |
| 0x01 | Action (c) | Action (a) |
| 0x02 | Action (a) | Action (e) |
| 0x03 | Action (f) (VID investigation) | Action (a) |
| 0x04 | Action (f) (VID investigation) | Action (e) |
| 0x05 | Action (g) (VID investigation) | Action (a) |
| 0x06 | Action (g) (VID investigation) | Action (e) |
| 0x07 | Action (f) (user priority investigation) | Action (a) |
| 0x08 | Action (f) (user priority investigation) | Action (e) |
| 0x09 | Action (g) (user priority investigation) | Action (a) |
| 0x0A | Action (g) (user priority investigation) | Action (e) |
| 0x0B | Action (f) (TCI investigation) | Action (a) |
| 0x0C | Action (f) (TCI priority investigation) | Action (e) |
| 0x0D | Action (g) (TCI investigation) | Action (a) |
| 0x0E | Action (g) (TCI investigation) | Action (e) |

## Extended MAC Bridge Filtering Scheme

### MAC Bridge Port Filter Preassign Table

This managed entity provides an alternate approach to address filtering than that supported in ITU-T G.983.2 [2] through MACBridgePortFilterTableData.  This alternate approach is useful when all the groups of addresses are stored beforehand in the line card, and this managed entity is used to designate which groups are valid or invalid for filtering.  The MAC addresses and Ether types for various protocols are provided in Appendix III. Instances of this managed entity are created/deleted autonomously after creation/deletion of a Subscriber Line Card ME of Ethernet type in which all groups of addresses are preassigned and stored in the card.

Relationships

This managed entity is associated with one instance of a MACBridgePortConfigurationData managed entity.

Attributes

| Managed Entity id: | This attribute provides a unique number for each instance of this managed entity. The assigned number is the same as the id of the MACBridgePortConfigurationData with which this MAC Bridge Port Filter Preassign Table instance is associated. (R) (mandatory) (2 bytes) |
| --- | --- |
| IPv4MulticastFiltering: | This attribute indicates whether IPv4Multicast Ether types are forwarded (value 0x00) or filtered (value 0x01). Initial value is 0x00. (R,W) (mandatory) (1 byte) |
| IPv6MulticastFiltering: | This attribute indicates whether Ipv6Multicast Ether types are forwarded (value 0x00) or filtered (value 0x01). Initial value is 0x00. (R,W) (mandatory) (1 byte) |
| IPv4BroadcastFiltering: | This attribute indicates whether IPv4Broadcast Ether types are forwarded (value 0x00) or filtered (value 0x01). Initial value is 0x00. (R,W) (mandatory) (1 byte) |
| RARPFiltering: | This attribute indicates whether RARP Ether types are forwarded (value 0x00) or filtered (value 0x01). Initial value is 0x00. (R,W) (mandatory) (1 byte) |
| IPXFiltering: | This attribute indicates whether IPX Ether types are forwarded (value 0x00) or filtered (value 0x01). Initial value is 0x00. (R,W) (mandatory) (1 byte) |
| NetBEUIFiltering: | This attribute indicates whether NetBEUI Ether types are forwarded (value 0x00) or filtered (value 0x01). Initial value is 0x00. (R,W) (mandatory) (1 byte) |
| AppleTalkFiltering: | This attribute indicates whether AppleTalk Ether types are forwarded (value 0x00) or filtered (value 0x01). Initial value is 0x00. (R,W) (mandatory) (1 byte) |
| BridgeManagement InformationFiltering: | This attribute indicates whether BridgeManagementInformation Ether types are forwarded (value 0x00) or filtered (value 0x01). Initial value is 0x00. (R,W) (mandatory) (1 byte) Note that some MAC addresses should not be handled, considering the following rules of IEEE 802.1D [5]: (1) Addresses from 0x0180C2000000 to 0x0180C200000F are reserved; (2) Addresses from 0x0180C2000020 to 0x0180C20002F are used for GARP Application Address. |
| ARPFiltering: | This attribute indicates whether ARP Ether types are forwarded (value 0x00) or filtered (value 0x01). Initial value is 0x00. (R,W) (mandatory) (1 byte) |
| PPPoEFiltering: | This attribute indicates whether PPPoE Ether types are forwarded (value 0x00) or filtered (value 0x01). Initial value is 0x00. (R,W) (mandatory) (1 byte) |

Actions

| Set: | Set one or more attributes. |
| --- | --- |

Notifications

| None. |  |
| --- | --- |

## Management support for video interface

### Physical Path Termination Point Video UNI

This managed entity represents the point at the video UNI in the ONT where physical paths terminate and physical path level functions are performed.

An instance of this managed entity shall be automatically created/deleted by the ONT upon creation/deletion of a Subscriber Line Card of Video type.

Relationships

One or more instances of this managed entity shall be contained in an instance of a Subscriber Line Card managed entity classified as Video type.

Attributes

| Managed Entity id: | This attribute provides a unique number for each instance of this managed entity. This 2-byte number is directly associated with the physical position of the UNI. The first byte is the slot id (defined in 7.1.3/G.983.2). If the UNI is integrated, this value is 0x00. The second byte is the port id with value range from 0x01 to 0xFF (1 to 255); 0x01 is used for the leftmost/lowest port on a Subscriber Line Card, 0x02 is used for the next right/upper port, and so forth. (R) (mandatory) (2 bytes) |
| --- | --- |
| Administrative State: | This attribute is used to activate (unlock: value 0x00) and deactivate (lock: value 0x01) functions performed by instances of this managed entity. Selection of a default value for this attribute is outside the scope of this document, as it is normally handled through supplier-operator negotiations. (R,W) (mandatory) (1 byte) |
| Operational State: | This attribute indicates whether or not this managed entity is capable of performing its task. The operational state reflects the perceived ability to receive or to generate a valid signal. Valid values are enabled (0x00) and disabled (0x01). (R) (optional) (1 byte) |
| ARC: | This attribute is used to control alarm reporting from this managed entity. Valid values are “off” (alarm reporting allowed immediately) and “on” (alarm reporting inhibited). Upon initial installation and provisioning of the ONT, this attribute may be set to “on” or “off” for the time interval specified by “ARCInterval”. Similarly, this attribute may be set to “off”. If the attribute is set to “on”, then alarm reporting is inhibited until this managed entity detects a valid signal for the time interval specified by “ARCInterval”. (R, W) (optional) (1 byte) |
| ARCInterval: | This attribute provides a provisionable length of time. Units are given in minutes. (R, W) (optional) (1 byte) |

Actions

| Get: | Get one or more attributes. |
| --- | --- |
| Set: | Set one or more attributes. |

Notifications

| Attribute value change: | This notification is used to report autonomous changes of attributes of this managed entity. The notification shall identify its new value. The AVC list is given in Table 9/G.983.8. |
| --- | --- |
| Alarm: | This notification is used to notify the management system when a failure has been detected or cleared. Both ONT and OLT should know the alarm list used by this entity. The alarm list for this entity is given in Table 10/G.983.8. |

Table 9/G.983.8  AVC list for Physical Path Termination Point Video UNI

| Number | Attribute Value Change | Description |
| --- | --- | --- |
| 1 | N/A |  |
| 2 | OpState | Operational state of video UNI |
| 3-16 | Reserved | Reserved for AVCs of vendor-specific attributes |

Table 10/G.983.8  Alarms list for Physical Path Termination Point Video UNI

| Number | Event | Description |
| --- | --- | --- |
| 0 | Video-LOS | No signal at the video UNI |
| 1-255 | Reserved | Reserved for vendor-specific alarms |

### Physical Path Termination Point Video ANI

This managed entity represents the point at the video ANI in the ONT where physical paths terminate and physical path level functions are performed.

An instance of this managed entity shall be automatically created/deleted by the ONT upon creation/deletion of a Subscriber Line Card of Video type.

Relationships

One or more instances of this managed entity shall be contained in an instance of a Subscriber Line Card managed entity classified as Video type.

Attributes

| Managed Entity id: | This attribute provides a unique number for each instance of this managed entity. There is only one instance, and it has the number 0x0000. (R) (mandatory) (2 bytes) |
| --- | --- |
| Administrative State: | This attribute is used to activate (unlock: value 0x00) and deactivate (lock: value 0x01) functions performed by instances of this managed entity. Selection of a default value for this attribute is outside the scope of this document, as it is normally handled through supplier-operator negotiations. (R,W) (mandatory) (1 byte) |
| Operational State: | This attribute indicates whether or not this managed entity is capable of performing its task. The operational state reflects the perceived ability to receive or to generate a valid signal. Valid values are enabled (0x00) and disabled (0x01). (R) (optional) (1 byte) |
| ARC: | This attribute is used to control alarm reporting from this managed entity. Valid values are “off” (alarm reporting allowed immediately) and “on” (alarm reporting inhibited). Upon initial installation and provisioning of the ONT, this attribute may be set to “on” or “off” for the time interval specified by “ARCInterval”. Similarly, this attribute may be set to “off”. If the attribute is set to “on”, then alarm reporting is inhibited until this managed entity detects a valid signal for the time interval specified by “ARCInterval”. (R, W) (optional) (1 byte) |

| ARCInterval: | This attribute provides a provisionable length of time. Units are given in minutes. (R, W) (optional) (1 byte) |
| --- | --- |
| FrequencyRangeLow: | This attribute indicates the lower of the possibly two frequency ranges supported. Different frequency ranges are indicated by code points, as given below: 0 indicates no low band is supported; 1 indicates 50-550 MHz is supported; 2 indicates 50-750 MHz is supported; 3 indicates 50-870 MHz is supported; 4..255 are reserved for future use. (R) (mandatory) (1 byte) |
| FrequencyRangeHigh: | This attribute indicates the higher of the two frequency ranges supported. Different frequency ranges are indicated by code points, as given below: 0 indicates no high band is supported; 1 indicates 550-750 MHz is supported; 2 indicates 550-870 MHz is supported; 3 indicates 950-2050 MHz is supported; 4 indicates 2150-3250 MHz is supported; 5 indicates 950-3250 MHz is supported; 6..255 are reserved for future use. (R) (mandatory) (1 byte) |
| SignalCapability: | This attribute indicates the capability of the ONT to measure the video signal level. Different capabilities are indicated by code points, as given below: 0 indicates no signal level capability is supported; 1 indicates total optical power level is supported; 2 indicates fixed frequency pilot tone power level is supported; 3 indicates total optical power level and fixed frequency pilot tone power level are supported; 4 indicates variable frequency pilot tone power level is supported; 5 indicates total optical power level and variable frequency pilot tone power level are supported; 6..255 are reserved for future use. (R) (mandatory) (1 byte) |

| OpticalSignalLevel: | This attribute indicates the current measurement of the total optical signal level. The unit of this attribute is dBW optical. If SignalCapability = 0, 2, or 4 then this attribute is undefined. If SignalCapability =1, 3, or 5, then this attribute describes the total optical power that is generating photocurrent on the receiver. (R) (optional) (1 byte) |
| --- | --- |
| PilotSignalLevel: | This attribute indicates the current measurement of the pilot signal level. The unit of this attribute is dBV at the RF video service port. If SignalCapability = 0 or 1, then this attribute is undefined. If SignalCapability =2, 3, 4, or 5, then this attribute describes the pilot signal level at the output of the video UNI. (R) (optional) (1 bytes) |
| SignalLevelMin: | This attribute indicates the minimum optical RF power per channel that will result in a CNR of 47 dBc for a channel of 4.5 MHz in bandwidth. The unit of this attribute is dBW optical. (R) (mandatory) (1 byte) |
| SignalLevelMax: | This attribute indicates the maximum optical RF power per channel that will result in a CTB of -57 dBc for an 80-channel ensemble of carriers. The units of the attributes are in dBW optical. (R) (mandatory) (1 byte) |
| PilotFrequency: | This attribute indicates the frequency of the pilot channel receiver. This unit of this attribute is Hz. If SignalCapability =0 or 1, this attribute is undefined; If SignalCapability =2 or 3, this attribute is functionally read only; If SignalCapability =4 or 5, this attribute is read-write. (R, W) (optional) (4 bytes) |

Actions

| Get: | Get one or more attributes. |
| --- | --- |
| Set: | Set one or more attributes. |

Notifications

| Attribute value change: | This notification is used to report autonomous changes of attributes of this managed entity. The notification shall identify its new value. The AVC list is given in Table 11/G.983.8. |
| --- | --- |
| Alarm: | This notification is used to notify the management system when a failure has been detected or cleared. Both ONT and OLT should know the alarm list used by this entity. The alarm list for this entity is given in Table 12/G.983.8. |

Table 11/G.983.8  AVC list for Physical Path Termination Point Video UNI

| Number | Attribute Value Change | Description |
| --- | --- | --- |
| 1 | N/A |  |
| 2 | OpState | Operational state of video ANI |
| 3-16 | Reserved | Reserved for AVCs of vendor-specific attributes |

Table 12/G.983.8  Alarms list for Physical Path Termination Point Video UNI

| Number | Event | Description |
| --- | --- | --- |
| 0 | Video-LOS | No signal at the video ANI |
| 1-255 | Reserved | Reserved for vendor-specific alarms |

## Management support for local craft terminal interface

### Physical Path Termination Point LCT UNI

This managed entity represents the point at the local craft terminal UNI in the ONT where physical paths terminate and physical path level functions are performed.

An instance of this managed entity shall be automatically created/deleted by the ONT upon creation/deletion of a Subscriber Line Card of LCT type.  However, this instance will not be reported during a MIB upload.

Relationships

One or more instances of this managed entity shall be contained in an instance of a Subscriber Line Card managed entity classified as LCT type.

Attributes

| Managed Entity id: | This attribute provides a unique number for each instance of this managed entity. This 2-byte number is directly associated with the physical position of the UNI. The first byte is the slot id (defined in 7.1.3/G.983.2). If the UNI is integrated, this value is 0x00. The second byte is the port id with value range from 0x01 to 0xFF (1 to 255); 0x01 is used for the leftmost/lowest port on a Subscriber Line Card, 0x02 is used for the next right/upper port, and so forth. (R) (mandatory) (2 bytes) |
| --- | --- |
| Administrative State: | This attribute is used to activate (unlock: value 0x00) and deactivate (lock: value 0x01) functions performed by instances of this managed entity. Selection of a default value for this attribute is outside the scope of this document, as it is normally handled through supplier-operator negotiations. (R,W) (mandatory) (1 byte) |

Actions

| Get: | Get one or more attributes. |
| --- | --- |
| Set: | Set one or more attributes. |

Notifications

| None. |  |
| --- | --- |

## Management support for ONUs

### ONT/ONU terminology

Throughout ITU-T G.983.2 [2], with the exception of the ONTB-PON Data managed entity description, the term ONT should generally be read as implying either ONT or ONU, whichever is appropriate for the particular instance.

### ONUB-PON

This managed entity represents the ONU as equipment.

An instance of this managed entity is automatically created by the ONU after initialization. After the creation of this managed entity, the associated attributes are updated according to the data within the ONU itself.

Relationships

All other managed entities in this Recommendation are related directly or indirectly to the ONUB-PON (or ONTB-PON) entity.

Attributes

| Managed Entity id: | This attribute provides a unique number for each instance of this managed entity. There is only one instance and it has the number 0x0000. (R) (mandatory) (2 bytes) |
| --- | --- |
| Vendor id: | This attribute identifies the vendor of the ONU. Upon autonomous instantiation, this attribute consists of all spaces. (R) (mandatory) (4 bytes) |
| Version: | This attribute identifies the version of the ONU as defined by the vendor. The printable value of "0" shall be used when version information is not available or applicable to the ONT being represented. Upon autonomous instantiation, this attribute consists of all spaces. (R) (mandatory) (14 bytes) |
| Serial Number: | The serial number is unique for each ONU. Note that the serial number of the ONU is already defined in ITU-T G.983.1 [1] and contains the vendor id and/or the version number. Upon autonomous instantiation, this attribute consists of all spaces. (R) (mandatory) (8 bytes) |
| Traffic management option: | This attribute identifies the upstream traffic management function implemented in the ONU. There are two options: 1) "Priority controlled upstream traffic" (0x00): the upstream traffic coming from the user is given a priority. 2) "Cell rate controlled upstream traffic" (0x01): the maximum upstream traffic of each individual connection is guaranteed. Note that the Traffic management option will not apply to downstream traffic. In other words, there is no need for a traffic descriptor for the downstream direction and downstream priority queues can be used. Upon autonomous instantiation, this attribute is set to 0x00. (R) (mandatory) (1 byte) |

| VP/VC cross-connection function option: | This attribute identifies the support of ATM VP or VC cross-connection management functions for the interworking connections to non-ATM UNIs. The value is set to 0x00 if ATM VP or VC cross-connection management functions are not modelled. The value is set to 0x01 if the ATM VP cross-connection management functions are modelled. The value is set to 0x02 if ATM VC cross-connection management functions are modelled. The default value of this attribute is 0x01. (R) (mandatory) (1 byte) |
| --- | --- |
| Battery backup: | This attribute provides a Boolean indication of whether or not the ONU supports battery backup. False will indicate that no battery is provisioned; true indicates that a battery is provisioned. Upon autonomous instantiation, this attribute is set to false. (R, W) (mandatory) (1 byte) |
| Administrative State: | This attribute is used to activate (unlock: value 0x00) and deactivate (lock: value 0x01) the functions performed by instances of this managed entity. Selection of a default value for this attribute is outside the scope of this document as it is normally handled through supplier-operator negotiations. (R, W) (mandatory) (1 byte) |
| Operational State: | This attribute indicates whether or not a managed entity is capable of performing its task. Valid values are enabled (0x00) and disabled (0x01). (R) (optional) (1 byte) |
| Equipment id: | This attribute may be used to identify the specific type of ONU. In North America, this may be used for the equipment CLEI code. (R) (optional) (20 bytes) |
| OMCC Version: | This attribute is used to identify the specific version of the OMCC protocol being used by the ONU. This is used to allow the OLT to manage a network with ONUs that support different OMCC versions. Valid values include 0x00 (2000 version) and 0x01 (2002 revised version). Future versions will be added sequentially. Default value is 0x00. (R) (optional) (1 byte) |
| Vendor Product Code: | This attribute is used to provide a vendor-specific product code for the ONT. (R) (optional) (2 bytes) |
| SecurityCapability: | This attribute is used to advertise the advanced security modes of the ONT. The following code points are defined: 0: No extra security features are supported; 1: AES encryption of the downstream payload is supported; 2..255: Reserved for future use. (R) (optional) (1 byte) |
| SecurityMode: | This attribute is used to select the advanced security mode for the ONT. Note that all secure VPs in an ONT must use the same security mode at any time. The following code points are defined: 0: Churning algorithm will be used; 1: AES algorithm will be used; 2..255: Reserved for future use. The default value for this attribute is 0. (R, W) (optional) (1 byte) |

Actions

| Get: | Get one or more attributes. |
| --- | --- |
| Set: | Set one or more attributes. |
| Reboot: | Reboot the ONU. |
| Test: | This action is used to initiate an ONU self test. Test outcome is “Pass” or “Fail.” |
| Synchronize Time: | This action is used to synchronize the start time of all Monitoring managed entities of the ONU with the reference time of the OLT and to reset the registers of the Monitoring managed entities. The effect of this action is that all counters of all Monitoring managed entities are set to 0x00 and restart counting. Also, the value of the Interval End Time attribute of the Monitoring managed entities is set to 0x00 and restarts counting. Note that no other OMCI action has the same effect: synchronization of the start time is not guaranteed at start-up or after a MIB reset command (optional). |

Notifications

| Attribute value change: | This notification is used to report autonomous changes to the attributes of this managed entity. The attribute value change notification shall identify the attribute changed and its new value. The list of AVCs for this managed entity is given in Table 13/G.983.8. |
| --- | --- |
| Alarm: | This notification is used to notify the managed system when a failure has been detected or cleared. Both ONT and OLT should know the alarm list used by this entity. The alarm list for this entity is given in Table 14/G.983.8. |
| Test result: | For the "Test result" event, notification is sent to the OLT via an alarm ONLY if the ME fails the autonomous self test. |

Table 13/G.983.8  AVC list for ONUB-PON

| Number | Attribute Value Change | Description |
| --- | --- | --- |
| 1-7 | N/A |  |
| 8 | OpState | Operational state of ONTB-PON |
| 9-16 | Reserved |  |

Table 14/G.983.8  Alarm list for ONUB-PON

| Number | Event | Description |
| --- | --- | --- |
|  | Alarm |  |
| 0 | EquipmentAlarm | A functional failure on an internal interface |
| 1 | PoweringAlarm | Loss of external power |
| 2 | BatteryMissing | Battery is provisioned but missing |
| 3 | BatteryFailure | Battery is provisioned and present but cannot recharge |
| 4 | BatteryLow | Battery is provisioned and present but its voltage is too low |
| 5 | PhysicalIntrusionAlarm | Applies if the ONT is supported with detection such as door or box open |
|  | Test result |  |
| 6 | ONUSelfTestFailure | ONU has failed autonomous self test |
| 7-255 | Reserved |  |

## Management support for VC cross-connections

### VC Network CTPB-PON

This managed entity is used to represent the termination of VC links on an ONT. An instance of the ATM VC Cross-Connection (i.e., VC MUX in ONT) managed entity may be used to relate two instances of the VC Network CTPB-PON managed entity for point-to-point cross-connection (multipoint cross-connection is for further study).

Instances of the VC Network CTPB-PON managed entity will be created on demand of the OLT:

•	as a consequence of action "create" on the VC Network CTPB-PON managed entity, or

•	as a consequence of action "create complete connection" on the ATM VC Cross-Connection managed entity.

Instances of the VC Network CTPB-PON managed entity will be deleted on demand of the OLT:

•	as a consequence of action "delete" on the VC Network CTPB-PON managed entity, or

•	as a consequence of action "delete complete connection" on the ATM VC Cross-Connection managed entity.

Notice that a VC Network CTPB-PON can be deleted only when no ATM VC Cross-Connection or Interworking VCC Termination Point is associated with it. It is the responsibility of the OLT to make sure that the VC Network CTPB-PON meets this condition at the time when the OLT requests to delete it.

Note that this managed entity aggregates connectivity functionality from the network view and alarms from the network element view as well as artefacts from trails.

Relationships

Zero or more instances of the VC Network CTPB-PON managed entity shall exist for each instance of the TC AdapterB-PON, PON TC Adapter or Interworking VCC Termination Point managed entity.

Relationship to Priority QueueB-PON /Traffic Descriptor Profile Pointer: see attribute definition.

Relationship to UPC Disagreement Monitoring History DataB-PON: one or zero implied in the managed entity id of UPC Disagreement Monitoring History DataB-PON.

This managed entity is related to the ATM VC Cross-Connection managed entity through the Termination Point ANI/UNI side attributes of the ATM VC Cross-Connection managed entity.

Attributes

| Managed Entity id: | This attribute provides a unique number for each instance of this managed entity. (R, Set-by-create) (mandatory) (2 bytes) |
| --- | --- |
| VPI Value: | This attribute identifies the VPI value associated with the VC link being terminated. (R, Set-by-create) (mandatory) (2 bytes) |

| VCI Value: | This attribute identifies the VCI value associated with the VC link being terminated. (R, Set-by-create) (mandatory) (2 bytes) |
| --- | --- |
| UNI/ANI Pointer : | This attribute associates the VC Network CTPB-PON with the ANI (i.e. PON interface) or a UNI. It points to the connected ANI/UNI instance id. Note: For the case using the multiplexing function of AAL2 (i.e. multiple instances of UNI are associated with a VC Network CTPB-PON instance), this attribute is assigned a special value: - 0x00XX will be used for pseudo slotIDs, - 0xXX00 will be used for pseudo portIDs. Therefore, 0x0000 will be used only for integrated interfaces (integrated type of ONT) that support multiple AAL2 functions. (R, Set-by-create) (mandatory) (2 bytes) |
| Direction: | This attribute specifies whether the VC link is used for UNI-to-ANI (value 0x01), ANI-to-UNI (value 0x02), or bi-directional (value 0x03) connection. (R, W, Set-by-create) (mandatory) (1 byte) |
| Priority Queue Pointer for downstream: | This attribute points to the instance of the Priority QueueB-PON used for this VC Network CTPB-PON in the downstream direction. Note that the value of this pointer is null when the VC Network CTPB-PON is at the ANI side. (R, Set-by-create) (mandatory) (2 bytes) |
| Priority Queue Pointer for upstream: | This attribute points to the instance of the Priority QueueB-PON used for this VC Network CTPB-PON in the upstream direction. It is used when the UNI/ANI pointer indicates an ANI instance id and the Traffic Management Option attribute in ONTB-PON is 0x00; this pointer is null otherwise. (R, Set-by-create) (mandatory) (2 bytes) |
| Traffic Descriptor Profile Pointer: | This attribute serves as a pointer to the instance of the Traffic Descriptor Profile managed entity that contains the traffic parameters used for this VC Network CTPB-PON. This attribute is used when the Traffic Management Option attribute in ONTB-PON is 0x01. It applies to the UNI side VC Network CTPB-PON if UPC is used. In this case, this pointer points to a Traffic Descriptor managed entity. When traffic shaping is used, it applies to the ANI side VC Network CTPB-PON. In this case, this pointer points to a Traffic Descriptor Profile managed entity and the Priority Queue Pointer for the upstream attribute is null. (R, Set-by-create) (optional) (2 bytes) See also Appendix IV.G.983.2. |

Actions

| Create: | Create an instance of this managed entity. |
| --- | --- |
| Delete: | Delete an instance of this managed entity. |
| Get: | Get one or more attributes of this managed entity. |
| Set: | Set one or more attributes of this managed entity. |

Notifications

| Alarm: | This notification is used to notify the management system for the ATM Layer Management Indication (LMI) when an alarm has been detected or cleared. The OLT should know the alarm list used by this entity. The alarm list for this entity is given in Table 15/G.983.8. See also Appendix III/G.983.2. |
| --- | --- |

Table 15/G.983.8  Alarm list for VC Network CTPB-PON

| Number | Alarm | Description |
| --- | --- | --- |
| 0 | VC-AIS-LMIR | VC-AIS receiving indication (optional) |
| 1 | VC-RDI-LMIR | VC-RDI receiving indication (optional) |
| 2 | VC-AIS-LMIG | VC-AIS generation indication (optional) |
| 3 | VC-RDI-LMIG | VC-RDI generation indication (optional) |
| 4 | Segment Loss of Continuity | Loss of continuity is detected when the VC Network CTPB-PON is a segment end point (optional) |
| 5 | End-to-End Loss of Continuity | Loss of continuity is detected when the VC Network CTPB-PON supports an Interworking VCC Termination Point (optional) |
| 6-255 | Reserved |  |

### ATM VC Cross-Connection

For point-to-point ATM VC Cross-Connections, this managed entity is used to represent the cross- connect relationship between two VC Network CTPB-PONs.  For multipoint ATM VC Cross-Connections, which are optional, the use of this managed entity is for further study.

Instances of this managed entity shall be created and deleted by the OLT based on ATM connection set up.

Relationships

Zero or more instances of the ATM VC Cross-Connection managed entity shall exist for each instance of the ONTB-PON  managed entity.

Attributes

| Managed Entity id: | This attribute provides a unique number for each instance of this managed entity. This 2-byte number is directly associated with the instance identifier of the VC Network CTPB-PON at the ANI side of this ATM VC cross-connect. (R, Set-by-create) (mandatory) (2 bytes) |
| --- | --- |
| Termination Point ANI side: | This attribute identifies the instance of the VC Network CTPB-PON managed entity that represents the cross-connected VC Network CTPB-PONs on the ANI side. (R, Set-by-create) (mandatory) (2 bytes) |
| Termination Point UNI side: | This attribute identifies the instance of the VC Network CTPB-PON managed entity that represents the cross-connected VC Network CTPB-PONs on the UNI side. (R, Set-by-create) (mandatory) (2 bytes) |
| Operational State: | This attribute indicates whether or not this managed entity is capable of performing its task. The operational state reflects the perceived ability to receive or to generate a valid signal. Valid values are enabled (0x00) and disabled (0x01). (R) (optional) (1 byte) |
| Administrative State: | This attribute is used to “unlock” (value 0x00) and “lock” (value 0x01) the functions performed by instances of this managed entity. (R, W, Set-by-create) (mandatory) (1 byte) |

Actions

| Create: | Create an instance of this managed entity. |
| --- | --- |
| Delete: | Delete an instance of this managed entity. |
| Create complete connection: | Create two instances of the VC Network CTPB-PON managed entity (ANI-side and UNI-side) and one instance of the ATM VC Cross-Connection managed entity. |
| Delete complete connection: | Delete two instances of the VC Network CTPB-PON managed entity (ANI-side and UNI-side) and one instance of the ATM VC Cross-Connection managed entity. |
| Get: | Get attributes of this managed entity. |
| Get complete connection: | Get all attributes of a connection; this holds the attributes of two instances of the VC Network CTPB-PON managed entity (ANI-side and UNI-side) and the attributes of the corresponding ATM VC Cross-Connection managed entity. |
| Set: | Set one or more attributes. |

Notifications

| Attribute value change: | This notification is used to report autonomous changes of attributes of this managed entity. The notification shall identify its new value. The list of AVCs for this managed entity is given in Table 16/G.983.8. |
| --- | --- |

Table 16/G.983.8 AVC list for ATM VC Cross-Connection

| Number | AVC | Description |
| --- | --- | --- |
| 1 | N/A |  |
| 2 | N/A |  |
| 3 | OpState | Operational state |
| 4 | N/A |  |
| 5-16 | Reserved |  |

### VC PM History Data

This managed entity is used to collect and report performance monitoring data associated with a VCC for the last completed 15-minute interval.  The instances of this managed entity are created and deleted on request of the OLT.

Relationships

Zero or more instances of this managed entity may exist for each instance of the VC Network CTPB-PON managed entity.

Attributes

| Managed Entity id: | This attribute provides a unique number for each instance of this managed entity. The assigned number is the same as the Managed Entity id of the corresponding VC Network CTPB-PON. (R, Set-by-create) (mandatory) (2 bytes) |
| --- | --- |
| Interval End Time: | This attribute identifies the most recently finished 15-minute interval. It is a cyclic counter (modulo 0xFF (256)) that is incremented each time a new interval is finished and the actual counters are updated. The value of this attribute is 0x00 during the first 15-minute interval that starts with the reception of the “synchronize time” action. The value is 0x01 during the first period after this, and so on. If this managed entity is created after the reception of the “synchronize time” action, the value of this attribute is set equal to the number of the last completed interval. The actual counters of this managed entity start counting directly. (R) (mandatory) (1 byte) |
| Threshold DataB-PON id: | This attribute provides a pointer to an instance of the Threshold Data B-PON managed entity that contains the threshold values for the performance monitoring data collected by this managed entity. (R, W, Set-by-create) (mandatory) (2 bytes) |
| Lost C=0+1 Cells: | This attribute measures background cell loss. It cannot distinguish between cells lost because of header bit errors, ATM-level header errors, cell policing, or buffer overflows. It records only loss of information independent of the priority of the cell. If the actual counter saturates, it remains on its maximum value. (R) (mandatory) (2 bytes) |
| Lost C=0 Cells: | This attribute measures background cell loss. It cannot distinguish between cells lost because of header bit errors, ATM-level header errors, cell policing, or buffer overflows. It records only loss of high priority cells. If the actual counter saturates, it remains on its maximum value. (R) (mandatory) (2 bytes) |
| Misinserted Cells: | This attribute is used to measure occurrences of when a cell is misrouted to an active VC that is being monitored. If the actual counter saturates, it remains on its maximum value. (R) (mandatory) (2 bytes) |
| Transmitted C=0+1 Cells: | This attribute provides a count of all cells that are originated at a monitored connection by the transmitting end point (i.e., backward reporting is assumed). (R) (mandatory) (5 bytes) |
| Transmitted C=0 Cells: | This attribute provides a count of all high priority cells that are originated at a monitored connection by the transmitting end point (i.e., backward reporting is assumed). (R) (mandatory) (5 bytes) |

| Impaired Block: | This severely errored cell block counter will be incremented whenever one of the following events takes place: the number of misinserted cells exceeds Mmisinserted, the number of bipolar violations exceeds Merrored, or the number of lost cells exceeds Mlost. The values for Mmisinserted, Merrored, and Mlost are set based on vendor-operator negotiation. (R) (mandatory) (2 bytes) |
| --- | --- |

Actions

| Create: | Create an instance of this managed entity. |
| --- | --- |
| Delete: | Delete an instance of this managed entity. |
| Get: | Get one or more attributes. |
| Set: | Set one or more attributes. |

Notifications

| Threshold Crossing Alert: | This notification is used to notify the management system when a threshold crossing alert (TCA) is detected or cleared. The TCA change notification “on” will be sent at the crossing of the threshold; the TCA change notification “off” will be sent at the end of the 15 min period since that is when the counters are reset to 0x00. Both ONT and OLT should know the event list used by this entity, given in Table 17/G.983.8. |
| --- | --- |

Table 17/G.983.8  Alarm list for VC PM History Data

| Number | Event | Description | Threshold Data counter #* |
| --- | --- | --- | --- |
|  | Threshold Crossing Alert |  |  |
| 0 | Lost CLP=0+ 1 Cells | Exceeds threshold | 1 |
| 1 | Lost CLP=0 Cells | Exceeds threshold | 2 |
| 2 | Misinserted Cells | Exceeds threshold | 3 |
| 3 | Impaired Blocks | Exceeds threshold | 4 |
| 4-255 | Reserved |  |  |

*This numbering is used with the associated Threshold Data B-PON managed entity.  Threshold Data counter 1 indicates the 1st thresholded counter, etc.

# Management support for additional Ethernet performance monitoring

    - Ethernet Performance Monitoring History Data 2

This managed entity contains the last completed 15-minute interval collected statistic data for an Ethernet interface. The statistic data value is only updated at the end of each period.

Instances of this managed entity are created/deleted by the OLT after an instance of Physical Path Termination Point Ethernet UNI managed entity is created/deleted.

Relationships

One instance of this Ethernet Performance Monitoring History Data 2 managed entity can exist for each instance of the Physical Path Termination Point Ethernet UNI.

Attributes

| Managed Entity id: | This attribute provides a unique number for each instance of this managed entity. This 2-byte number is directly associated with the id of the Physical Path Termination Point Ethernet UNI. (R, Set-by-create) (mandatory) (2 bytes) |
| --- | --- |
| Interval End Time: | This attribute identifies the most recently finished 15-minute interval. It is a cyclic counter (modulo 0xFF (256)) that is incremented each time a new interval is finished and the statistic data values are updated. The value of this attribute is 0x00 during the first 15-minute interval that starts with the reception of the "synchronize time" action. The value is 0x01 during the first period after this, and so on. If this managed entity is created after the reception of the "synchronize time" action, the value of this attribute is set equal to the number of the last completed interval. The actual counters of this managed entity start counting directly. The statistic data value is updated at the end of the interval. (R) (mandatory) (1 byte) |
| Threshold Data B-PON id: | This attribute provides a pointer to an instance of the Threshold Data APON managed entity that contains the threshold values for the performance monitoring data collected by this managed entity. (R, W, Set-by-create) (mandatory) (2 bytes) |
| PPPoEFilteredFrame Counter: | This attribute provides a count of the number of frames that have been discarded due to PPPoE filtering. Default value is 0x00. (R) (mandatory) (4 bytes) |

Actions

| Create: | Create an instance of this managed entity. |
| --- | --- |
| Delete: | Delete an instance of this managed entity. |
| Get: | Get one or more attributes. |
| Get Current Data: | This action returns the current value of one or more actual counters associated with performance monitoring attributes and the value of the Interval End Time attribute representing the interval in which the request is made. The values in the specific counters would be reset at the end of the interval. (Note: “Get” returns the statistical data stored in the attribute values; “Get current data” returns the real-time value of the actual counters associated with those attributes.) Support of this action is optional. |
| Set: | Set one or more attributes. |

Notifications

| Threshold Crossing Alert: | This notification is used to notify the management system when a threshold crossing alert (TCA) is detected or cleared. The TCA change notification "on" will be sent at the crossing of the threshold by the actual counter; the TCA change notification "off" will be sent at the end of the 15 min period since that is when the actual counters are reset to 0x00. The event list for this entity is given in Table 18/G.983.8. Table 18/G.983.8 replaces Table 13-a/G.983.2. Modifications from Table 13-a/G.983.2 are indicated by underlined text. |
| --- | --- |

Table 18/G.983.8  Alarm list for Ethernet Performance Monitoring History Data 2

| Number | Event | Description | Threshold Data counter #* |
| --- | --- | --- | --- |
|  | Threshold Crossing Alert |  |  |
| 0 | PPPoEFilteredFrameCounter | Exceeds threshold | 1 |
| 1-255 | Reserved |  |  |

*This numbering is used with the associated Threshold Data B-PON managed entity.  Threshold Data counter 1 indicates the 1st thresholded counter, etc.

# ONT Management and Control Channel (OMCC)

See clause 8/G.983.2.

# ONT Management and Control Protocol

See clause 9/G.983.2.  Modifications to this clause are provided in the sections below.

## Message types

- To support VC cross-connections, message types 5 and 7 in Table 20/G.983.2 must be modified.  Table 19/G.983.8 gives the modified message types, where underlined text identifies the modifications.

Table 19/G.983.8  OMCI message types

| MT | Type | Purpose | AK | Inc MIB data sync. |
| --- | --- | --- | --- | --- |
| 5 | Create complete connection | Create an ATM VP Cross-Connection and two associated VP Network CTPB-PONs or create an ATM VC Cross-Connection and two associated VC Network CTPB-PONs | yes | yes |
| 7 | Delete complete connection | Delete an ATM VP Cross-Connection and two associated VP Network CTPB-PONs or delete an ATM VC Cross-Connection and two associated VC Network CTPB-PONs | yes | yes |

## Managed Entity identifiers

The ONT management and control protocol cell format is defined in Recommendation G.983.2.  As new managed entities are introduced into the OMCI specifications, the managed entity identifier that is used in the message identifier field shall be defined. Table 20/G.983.8 gives the class values for the new managed entities. The class values for existing managed entities are found in Table 21/G.983.2.

Table 20/G.983.8 – 	Managed entity identifiers

| Managed entity class value | Managed entity |
| --- | --- |
| 67 | IP Port Configuration Data |
| 68 | IP Router Service Profile |
| 69 | IP Router Configuration Data |
| 70 | IP Router PM History Data 1 |
| 71 | IP Router PM History Data 2 |
| 72 | ICMP PM History Data1 |
| 73 | ICMP PM History Data 2 |
| 74 | IP Route Table |
| 75 | IP Static Routes |
| 76 | ARP Service Profile |
| 77 | ARP Configuration Data |
| 78 | VLAN Tagging Operation Configuration Data |
| 79 | MAC Bridge Port Filter Preassign Table |
| 80 | Physical Path Termination Point ISDN UNI |
| 81 | (Reserved for Physical Path Termination Point HPNA UNI) |
| 82 | Physical Path Termination Point Video UNI |
| 83 | Physical Path Termination Point LCT UNI |
| 84 | VLAN Tagging Filter Data |
| 85 | ONUB-PON |
| 86 | ATM VC Cross-Connection |
| 87 | VC Network CTPB-PON |
| 88 | VC PM History Data |
| 89 | Ethernet Performance Monitoring History Data 2 |
| 90 | Physical Path Termination Point Video ANI |
| 91..255 | Reserved |

For other specifications, refer to clause 9/G.983.2.

APPENDIX I

OMCI common mechanisms and services

This appendix describes the common mechanisms and services of the OMCI that are related to IP Router function and VC cross-connections.  For all other common mechanisms and services, refer to Appendix I/G.983.2.

  - Common mechanisms

The following common mechanisms shall be added to the list given in clause I.1/G.983.2.

p)	IP Router service connection set-up;

q)	IP Router service connection tear-down;

r)	Addition of entities to IP Static Routes; and

s)	Removal of entities from IP Static Routes.

These common mechanisms will be explained by the use of scenario diagrams.

  - IP Router service connection set-up

Figure I.1/G.983.8 shows the scenario for IP Router service connection set-up for an ONT with cross-connect functionality. For the case that an ONT does not model cross-connect function, the Interworking VCC Termination Point is directly associated to the VP Network CTPB-PON on the ANI side.

Note that the AAL5 profile can be shared among multiple Interworking VCC Termination Points. No creation of profiles is needed if the Interworking VCCTP points to an existing profile.
Moreover, the IP Router Service Profile and ARP Service Profile can be shared among multiple IP Port Configuration Data managed entities, so no creation of profiles is needed if the IP Port Configuration Data points to an existing profile.

The OLT also may want to create corresponding History Data managed entities for the connection.

Figure I.1/G.983.8  Connection set-up for IP Router service connection

### IP Router service connection tear-down

The following scenario, Figure I.2/G.983.8, depicts IP Router service connection tear-down for an ONT with cross-connect functionality. For the case that an ONT does not model cross-connect function, the Interworking VCC Termination Point is directly associated to the VP Network CTPB-PON on the ANI side.

Note that the AAL5 profile can be shared among multiple Interworking VCC Termination Points. If there are more Interworking VCC Termination Points associated with this profile managed entity, the OLT may not request to delete it. This holds also for the ATM connection used: if more Interworking VCC Termination Points are associated to this connection (i.e. VP Network CTPB-PON), the ATM connection cannot be deleted.  Moreover, the IP Router Service Profile and ARP Service Profile can be shared among multiple IP Port Configuration Data managed entities. If there are more IP Port Configuration Data managed entities associated with these profile managed entities, the OLT may not request to delete them.

If applicable, the OLT can delete the corresponding History Data managed entities as well.

Figure I.2/G.983.8  Connection tear-down for IP Router service connection

### Addition of entries to IP Static Routes

The following scenario, Figure I.3/G.983.8, depicts addition of entities to the IP Static Routes managed entity for an ONT.

Figure I.3/G.983.8  Addition of entities to the IP Static Routes

    - Removal of entries from IP Static Routes

The following scenario, Figure I.4/G.983.8, depicts the removal of entries from the IP Static Routes managed entity for an ONT.

Figure I.4/G.983.8  Removal of entries from IP Static Routes

  - Common Services

    - Update common services table

Common services f) and g) in clause I.2/G.983.2 need to be updated as shown below to support VC cross-connections.

- ATM VP Cross-Connection or ATM VC Cross-Connection setup;

- ATM VP Cross-Connection or ATM VC Cross-Connection breakdown.

    - Update ATM service set-up and take-down

Clauses I.2.7-I.2.8/G.983.2 provide descriptions for ATM service set-up and take-down using VP cross-connections. These descriptions can be extended for use with VC cross-connections as well.  For ATM service set-up and take-down using VC cross-connections, replace “VP Network CTPB-PONs” with “VC Network CTPB-PONs” and “ATM VP Cross-Connection” with “ATM VC Cross-Connection”.

    - Update common mechanism scenarios

Clauses I.2.9-I.2.12/G.983.2 provide common mechanism scenarios involving VP cross-connections.  These scenarios can be extended for use with VC cross-connections as well. To do so, replace the term “VP Network CTPB-PON” with “VP Network CTPB-PON or VC Network CTPB-PON”.

APPENDIX II

OMCI Message set

  - Create Complete Connection

Bytes 13-16 of clause II.2.3/G.983.2 must be modified as indicated below to provide support for VC cross-connections.

| Field | Byte | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message contents | 13 |  |  |  |  |  |  |  |  | msb ani VP Network CTPB-PON or VC Network CTPB-PON instance |
|  | 14 |  |  |  |  |  |  |  |  | lsb ani VP Network CTPB-PON or VC Network CTPB-PON instance |
|  | 15 |  |  |  |  |  |  |  |  | msb uni VP Network CTPB-PON or VC Network CTPB-PON instance |
|  | 16 |  |  |  |  |  |  |  |  | lsb uni VP Network CTPB-PON or VC Network CTPB-PON instance |

  - Get Complete Connection Response

Bytes 14-17 of clause II.2.14/G.983.2 must be modified as indicated below to provide support for VC cross-connections.

| Field | Byte | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message contents | 14 |  |  |  |  |  |  |  |  | msb ani VP Network CTPB-PON or VC Network CTPB-PON instance |
|  | 15 |  |  |  |  |  |  |  |  | lsb ani VP Network CTPB-PON or VC Network CTPB-PON instance |
|  | 16 |  |  |  |  |  |  |  |  | msb uni VP Network CTPB-PON or VC Network CTPB-PON instance |
|  | 17 |  |  |  |  |  |  |  |  | lsb uni VP Network CTPB-PON or VC Network CTPB-PON instance |

APPENDIX III

MAC addresses and Ether types

Table III.1/G.983.8 –	MAC address and Ether type for various protocols

| # | Protocol | MAC address | Ether type | Standard |
| --- | --- | --- | --- | --- |
| 1 | IPv4 Multicast | 0x01005E000000 ~ 0x01005E7FFFFF | - | RFC1700 [App IV-3] |
| 2 | IPv6 Multicast | 0x333300000000 ~ 0x3333FFFFFFFF | - | RFC2464 [App IV-4] |
| 3 | IPv4 Broadcast | 0xFFFFFFFFFFFF | 0x0800 | RFC1700 [App IV-3] |
| 4 | RARP | 0xFFFFFFFFFFFF | 0x8035 | RFC1700 [App IV-3] |
| 5 | IPX | 0xFFFFFFFFFFFF | 0x8137 | RFC1700 [App IV-3] |
|  |  | 0x09001BFFFFFF, 0x09004E000002 | - |  |
| 6 | NetBEUI | 0x030000000001 | - |  |
| 7 | AppleTalk | 0xFFFFFFFFFFFF | 0x809B, 0x80F3 | RFC1700 [App IV-3] |
|  |  | 0x090007000000 ~ 0x0900070000FC, 0x090007FFFFFF | - |  |
| 8 | Bridge Management Information | 0x0180C2000000 ~ 0x0180C20000FF | - | IEEE802.1D [5] |
| 9 | ARP | 0xFFFFFFFFFFFF | 0x0806 | RFC1700 [App IV-3] |
| 10 | PPPoE Broadcast | 0xFFFFFFFFFFFF | 0x8863 | RFC2516 [App IV-5] |

APPENDIX IV

Bibliography

[App.IV-1]	IETF RFC 815 (1982), IP Datagram Reassembly Algorithms.

[App IV-2]	IETF RFC 1213 (1991), Management Information Base for Network Management of TCP/IP-based internets: MIB-II.

[App IV-3]		IETF RFC 1700 (1994), Assigned Numbers.

[App IV-4] 	IETF RFC 2464 (1998), Transmission of IPv6 Packets over Ethernet Networks.

[App  IV-5]	IETF RFC 2516 (1999), A Method for Transmitting PPP Over Ethernet (PPPoE).

___________________